# Homework 1: Road Vehicle Performance

### Additional Required Work

I chose a 2003 Honda Element four-wheel drive EX model vehicle for this assignment.  Figure 1 is a plot of its braking distance vs. initial speed for both theoretical minimum braking distance and practical braking distance used in design.  Supporting equations and assumptions are shown on the following page.

Figure 1: Comparison of theoretical braking distance for the 2003 Honda Element 4WD EX and practical braking distance used in design.

Braking distance:  S=γbW2gKaln1+KaV12ηbμW+frlW±Wsinθg

| Where: | S | = | theoretical minimum braking distance (ft.) |
| --- | --- | --- | --- |
|  | γb | = | mass factor (given) |
|  | W | = | weight of vehicle (in lbs.). Obtained from: http://www.hondacars.com/models/specifications.asp?ModelName=Element |
|  | g | = | acceleration due to gravity (32.2 ft/sec2) |
|  | Ka | = | air resistance term (more below) |
|  | V | = | Vehicle speed in ft/sec |
|  | ηb | = | Braking efficiency (given) |
|  | μ | = | Coefficient of road adhesion (given) |
|  | frl | = | Coefficient of rolling resistance |
|  | θg | = | Angle of incline (in this case, θ = 0 for a flat grade) |

Air resistance:  Ka=ρ2CDAf

| Where: | Ka | = | air resistance term (more below) |
| --- | --- | --- | --- |
|  | ρ | = | Air density (slugs/ft3) |
|  | CD | = | Coefficient of drag. The Honda Element value was not readily obtainable so I selected a vehicle with a similar shape, the Hummer H2, and used its drag coefficient as an estimate of the Element’s. A value of 0.57 was obtained from: http://www.motortrend.com/roadtests/suv/112_0208_hummer/index1.html |
|  | Af | = | Frontal area, estimated by multiplying the vehicle width (71.5 inches) by the difference between the height (70.4 inches) and ground clearance (6.9 inches). Dimensions obtained from: http://www.hondacars.com/models/specifications.asp?ModelName=Element |

Practical braking distance for design: d=V230a32.2±G

| Where: | d | = | practical braking distance for design (ft.) |
| --- | --- | --- | --- |
|  | V | = | vehicle speed in miles/hour |
|  | a | = | deceleration (given as 11.2 ft/sec2) |
|  | G | = | grade, as a decimal |

Supporting Excel tables and calculations are shown on the next page.