# Homework 1: Road Vehicle Performance

### Textbook Assignment

| Problem | Value | Answer |
| --- | --- | --- |
| 2.2 | 10 | Final weight = 2210.45 lb |
| 2.7 | 10 | 185 ft-lb engine = 13.33 ft/sec2; 215 ft-lb engine = 14.15 ft/sec2 |
| 2.13 | 10 | 8.01 ft/sec2 |
| 2.14 | 10 | 219.9 mph |
| 2.25 | 10 | About 207 ft |
| 2.28 | 10 | You figure this one out |

### Additional Required Work (40 points)

Your final write-up should concise and to-the-point.  Key answers and assumptions should be clearly stated and supporting calculations should be included after the write-up and referenced in the write-up.

Part 1

Pick a vehicle of your choice.  Using Microsoft Excel (or a similar program) plot this vehicle’s theoretical braking distance (in feet) vs. speed (in mph) for speeds of 0 to 120 mph.  Assume there are two people in the vehicle.  Assume the following parameters:

- Braking efficiency = 95% for antilock brakes, 80% for non-antilock brakes

- Coefficient of road adhesion = 0.70

- Grade = 0% (flat)

- Air density = 0.0024 slugs/ft3

  - note: 1 slug = 1 lb force / (1 ft/s2 acceleration)

  - note also: 1 lb force = 1 slug·ft/s2 = 4.448 Newtons

- Mass factor = 1.04 (accounts for moments of inertia during braking)

This assignment will require you to look up the following vehicle characteristics:

- Coefficient of drag

- Frontal area = (height – ground clearance) × width

- Weight (unloaded and with two people in it)

Part 2

On the same graph, plot practical braking distance (in feet) vs. speed (in mph) for speeds of 0 to 120 mph.  Use the AASHTO braking distance formula:

| d=V230ag±G |  | V = vehicle speed in miles/hour a = deceleration rate in ft/sec2 (use 11.2 ft/sec2) g = acceleration due to gravity (use 32.2 ft/sec2) G = grade in decimal form (e.g., 6% = 0.06) |
| --- | --- | --- |