# Homework 2: Geometric Design & Pavement Design

### Textbook Assignment

| Number | Points | Answer |
| --- | --- | --- |
| 3.2 | 10 | (high pt., sta. = 340+58, el. = 1318.15 ft; PVC, sta = 337+50, el. = 1312 ft; PVT sta = 342+50, el. = 1315.75 ft) |
| 3.7 | 10 | 8.9 ft |
| 3.8 | 10 | The curve is adequate for the conditions. The criteria one could use are K = 206.5 (greater than required K = 151) or minimum L = 583.2 ft (less than actual L = 800) |
| 3.25 | 10 | PC sta = 2694+90, PT sta = 2704+68, safe speed = 59.26 mph |
| 3.27 | 10 | e = 0.413, D = 5.73, L = 523.6 ft, STAPC = 1122+42.05, STAPT = 1127+65.65 |
| 3.31 | 20 | You figure this one out |

### Additional Required Work (30 points)

Using the 1993 AASHTO equation and common assumptions, how many ESALs should the Interstate 5 rigid pavement under the 45th St. overpass in the southbound direction support? How many has it supported already?

Note: you must determine it’s structure and other information from http://wspms.ce.washington.edu/. This will require you to spend a bit of time figuring out how this application works. There are multiple ways to answer this question.