Lab 8: Spatial Analyst

A) Population Densities

This lab will make use of Raster files and the Spatial Analyst tools. Remember that a raster is a grid of information. The population density raster file given here has a pixel size of 100m by 100m.

First make sure the Spatial Analyst is on. Go to Tools > Extensions > Spatial Analyst. Make sure this is checked on. Then go to View > Toolbars > Spatial Analyst so it shows up on the screen.

“Spatial Analyst provides a broad range of powerful spatial modeling and analysis features previously not available to desktop users. It allows you to create, query, map, and analyze cell-based raster data and to perform integrated vector–raster analysis.”

-ESRI

Please see this site and review the statistical analyses that can be performed with Spatial Analyst.

- Calculate the population of the Shoreline, Lake Forest Park and Kenmore school district. Use zonal statistics. (Hint: it will be helpful to join the statistics to the zone table.)

Don’t forget to think of the statistics as being representative of pixels and that the data is population density (pop / Sq Mi). Yes there is a conversion factor you can figure out that will be helpful.

Once you click on zonal statistics you will need to realize that one input file is for a raster and another is an input for a vector file. You should be able to determine which files go where. Also check which field you will be processing on. No chart will be needed but might be interesting to look at. Once again remember units when looking at the statistics. With the stats joined it should be a simple matter of operations within Arc to calculate the population  or a matter of calculations with any other number crunching means you have.

B) Trail Elevation

- Which trail systems can we hike that are over 1000 ft based on the elevation data that we currently have?

The units of the DEM are in feet.

Turn on Spatial Analyst in ArcMap

Tools > Extensions... > check Spatial Analyst > Close
then

View > Toolbars > check Spatial Analyst
The Spatial Analyst Toolbar displays and can be repositioned

To accomplish this task you will first make use of the raster calculator in the spatial analyst. First observe what raster file is in the drop down menu for the Spatial Analyst tool bar (if the toolbar is not open , go to view, toolbars). This should be set to the Elevation raster. Next click the menu for spatial analyst and go to Raster Calculator. The idea of the raster calculator is rather straight forward for simple concepts. The elevation raster has values in each pixel representing elevation and we wish to look at areas with elevations above a certain level. So with the raster calculator we simply double click the raster desired use the greater-than symbol and enter our desired elevation of interest.

This should provide us with a new raster file made up of two distinct values. We will change this raster file such that it will only provide us with data about the areas of elevation above the desired level. To do this we use the spatial analyst tool Reclassify.

Open the reclassify tool and make sure your input raster is set to the one which we just calculated. You should see something similar to this.

Since our map currently represents the 0 value as values that were below our selected elevation we will want to get rid of this data. So in the column listed as new values we will click on “1” and change it to “NoData”. Then select OK. The data should now have changed to be just the areas above our given elevation.

Next go to your toolbox > Conversion tools > From Raster > Raster to Polygon and change the raster file representing our elevation to a vector polygon file. From here you should have the knowledge of what you have previously done in this class to determine which trails we can hike in order to be above our given elevation.

Print a Layout of the trails that are above 1000 feet, the scale must be 1:10000. State the length of the trail that is above 1000 feet in the layout.