|  | # STATE OF WASHINGTON ## DEPARTMENT OF HEALTH ## DIVISION OF ENVIRONMENTAL HEALTH SERVICES # Tattoo, Permanent Makeup, and Body Piercing Inspection Checklist |  |  |
| --- | --- | --- | --- |
| KENNETH WARDEN, EHS, DIRECTOR |  | ##### 1234 Capitol Way, Olympia, Washington 98583-0111 ##### PH: 360-752-1111 FAX 360-752-9999 health@washington.wa.us |  |

Facility:__Creative Tattoos__________________                     Date:__February 20, 2003___________

#### Address: ___________________________

| ### Disease Control | Y | N |
| --- | --- | --- |
| Disinfectant used for sanitizing “critical” equipment (needles, bars) e.g., Cidex or Madacide; must be approved for Hepatitis B & HIV disinfection. | X |  |
| Household bleach (5.2%) may be used on “noncritical” surfaces at 1: 50 dil. or 1000 ppm (made fresh daily) | X |  |
| Autoclave—anaerobic spore count analysis is recommended monthly. | X |  |
| Sharps container is used for all sharp objects, including needles and razors. | X |  |
| Single-use stencils (unless sanitized in accordance with Health Department) are used. | X |  |
| Aseptic dispensing of petroleum jellies, soaps, and other products used in the application of stencils & tattoos | X |  |
| Autoclave methods: dry heat-320F>1 hour Or steam heat-250F>30 min @ 15 psi | ** |  |
| Non-disposable equipment, i.e., needle bars, needle caps, needle tubes, body piercing tubes & other contaminated instruments are stored safely prior to being cleaned and sterilized. | X |  |
| Before being sterilized, any used non-disposable piercing and tattooing components are cleaned in an ultrasonic cleaner. | X |  |
| Tattoo machine is disassembled & disinfected after each use. | X |  |
| Micropigmentation device (permanent cosmetics) is disassembled after each use: casing must be removed, washed with a small brush and disinfectant, and sterilized in an autoclave or cold sterilization solution unless disposable, single-use casings are used. | X |  |
| If sheets or towels are used, they are changed after each customer. | X |  |
| Body surfaces are washed prior to procedure (except eyelids). | X |  |
| Single use equipment, such as tongue depressors, gauze, rubber bands, plastic bags (for covering disinfectant bottles and tattoo machines during tattoo procedures) are stored in a clean, dry place. | X |  |
| Employees are vaccinated for Hepatitis B. | X |  |

| ## Structural | ### Y | N |
| --- | --- | --- |
| Floors, walls, ceiling—easily washable, impermeable | X |  |
| All procedure surfaces (chairs, benches)—easily cleaned and sanitized after each client | X |  |
| Handsink, with hot and cold running water, soap and disposable paper towels | X |  |
| Minimum of 45 square feet of floor space for each operator in facility | X |  |
| Dividers used in facilities with multiple stations | X |  |
| No food or drinks are consumed or stored in procedure area. | X |  |
| Supplies/Equipment | Y | N |
| Disposable, single-use needles are used. Note: Piercing guns are NOT permitted for any piercing except that of the ear lobe. | X |  |
| Disposable gloves are used exclusively. | X |  |
| Single-use pigment/ink wells are used exclusively. | X |  |
| Single-use razors are used exclusively. | X |  |
| Jewelry used in body piercing must be made of surgical implant-grade stainless steel, solid 14k or 18k yellow or white gold, niobium, titanium, or platinum, or a low-porosity plastic that has been properly sterilized in an autoclave prior to use. | X |  |
| Autoclave indicator tape or temperature-sensitive autoclave bags are used for autoclaving equipment. | X |  |
| Trash bags contain liners which are replaced daily. | X |  |
| Business Policies | Y | N |
| Tattooing is not performed on minors; piercing is performed on minors only with written parental consent. |  |  |
| Release forms are kept for 3 years. |  |  |
| Written SOP is in place for disinfecting facility daily. |  |  |
| Written SOP is in place for disinfecting workstation after each procedure. |  |  |
| Each employee has taken Blood Borne Pathogen Workshop. |  |  |
| Full sharps containers are properly disposed. |  |  |

Comments:  Inspection prompted by report from medical provider that patient tattooed here came down with Hepatitis B.  Inspection gave no indication of any current problems.  Parlor is clean, with appropriate procedures in place.  **Notice above autoclave emphasized need to re-start sterilization procedure if autoclave becomes unplugged.  I asked owner what prompted notice.  Owner indicated that machine had accidentally become unplugged on one occasion back in November 2002 and new employee may not have re-started procedure from beginning.  Owner has reorganized work space to avoid accidental unplugging.  Violation issued.

Inspected by: __JJ Jones____________