A Guide to Estimating Logging Costs

in the Pacific Northwest (PNW)

World Forest Institute

Portland, OR, USA

January 1997

Table of contents:

1.	Introduction

2.	Disclaimer

3.	Logging cost estimation - users guide

4.	Machine description

5.	Definitions and assumptions

5.1 	Time availability

5.2	Cost categories

5.3	Cost items

Ownership costs

Operating costs

Auxiliary (overhead) costs

5.4	Ownership period, residual value and repair reserve - criteria

5.5	Production units

5.6	Tree size

5.7	Harvest prescription

5.8	Yarding distance

5.9	Terrain class

5.10	Wages and benefits

5.11	Manual falling

5.12	Cable harvesting accessories

6.	Acknowledgments

Appendices (Logging Cost Estimates)

Tab 1	Common Factors

Tab 2	Cable yarding		- 40’ tower with stroke delimber

Tab 3	Cable yarding		- 40’ tower with manual delimbing

Tab 4	Cable yarding		- 50’ swing yarder

Tab 5	Cable yarding		- 70’ tower with stroke delimber

Tab 6	Cable yarding		- 70’ tower with manual delimbing

Tab 7	Cut-to-length harvesting

Tab 8	Helicopter yarding	- non pre-bunching

Tab 9	Helicopter yarding	- pre-bunching

Tab 10	Shovel logging

Tab 11	Skidder yarding	- wheel skidders with feller buncher

Tab 12	Skidder yarding	- wheel skidders with manual falling/delimbing

Tab 13	Skidder yarding	- track skidders with feller buncher

Tab 14	Skidder yarding	- track skidders with manual falling/delimbing

1.	Introduction

The Oregon Department of Forestry (ODF) and Federal Bureau of Land Management (BLM) enlisted the World Forest Institute (WFI) to develop an up-to-date method of estimating typical harvesting costs for a range of common logging conditions.  Both agencies, as public forest managers, have an obligation to ensure that standing timber is sold for a fair market value (or accredited for market value in land exchanges).  The cost of logging needs to be estimated as a component of determining the market value of standing timber.

As the appraisal process is generally conducted prior to the sale and harvest of any particular parcel of timber (or exchange of land), details such as the purchaser, logging system and specific machine combination are not necessarily known at the time of appraisal.

This guide to estimating logging costs is designed for the purposes of appraising timber sales and combines a cost analysis of owning and operating the various machines commonly used for logging in the PNW with typical production rates given a typical range of site and stand variables.  It thus represents the costs associated with common industry practice rather than the exact make-up of any single side.  It is recognized that, in practice, the logging system actually used depends largely upon machine availability and that logging systems are generally designed to provide maximum flexibility for a variety of logging conditions rather than best suited to the job currently at hand.

The guide is also designed to provide simple and practical use in the field.  Appraisers with a sound understanding of local stand and site conditions can use the guide to estimate logging costs for an individual site without having to undertake complex calculations or input additional detailed data.

Although aimed primarily at logging operations in public forests, it will also provide a useful resource for the private sector in analyzing operational costs and bid preparation.  Appraisers and planners may use the guideline to compare the cost of particular logging systems for a given timber sale and to consider the impact on logging cost (and ultimately stumpage value) of planning options such as sale layout (e.g. roading and reserve design) and bid requirements (e.g. specifying the logging system).

Common industry practice and typical production rates were established following interviews with a variety of logging contractors, industry representatives, machinery manufacturers and sales representatives, industry consultants and others.

Due to difficulties in obtaining detailed cost data through direct survey, the likely machine costs and a range of production rates have been used to enable estimation of logging costs.

The guide does not attempt to estimate production for a particular logging operation, but rather provides a range of possible production rates (based on estimates by logging companies) from which a user with field experience is able to select the most appropriate production level.

The cost estimations included are “on-truck” only and do not include the cost of additional works, such as road construction or silvicultural/rehabilitation treatment, normally not required as part of the logging operation.  Allowances for investment risk and profit margin have not been included.  In addition, the guide is not intended for direct use in contract or rate negotiations, although they may be used to assist operators in calculating their individual operating costs.

2.	Disclaimer

While the cost and production data contained within this guide is intended to reasonably represent typical logging operations in the PNW, it may not accurately reflect any specific logging operation.  As such, this guide should not be used to determine the exact cost of an individual logging operation.

3.	Logging cost estimation - users guide

3.1	General

This guideline to estimating the cost of logging in the PNW is designed to provide flexibility in use in the field and the opportunity for specific manual adjustment where required.

The ability to manually adjust the base data and assumptions made provides the user with some flexibility to reflect individual circumstance, although as most appraisals occur prior to the bidding process, the guideline is aimed at providing the user with a range of typical logging costs for each common yarding system, assuming the given average tree sizes.

The guideline does not attempt to estimate production, but provides an estimate of logging costs under various yarding systems for a range of typical daily production.  Field experience can then be used to refine the logging cost estimate according to an estimated specific level of average daily production, or the mid point of the typical production range may be selected as a representative figure where only a general approximation of the logging cost is required.

3.2	Data update

It is recommended that a master copy be kept for reference and to provide a base for future updates and refinements of the cost and production data.  Working copies of the guideline may then be used by appraisers and planners and adjusted/refined to suite local practices and conditions.

Updates of machine ownership and operating costs are recommended to be carried out on a regular basis (yearly) in order to provide the most accurate and current reflection of logging costs.  Similarly, it is advised that some resources be devoted on a regular basis to verifying likely production ranges for selected yarding systems and identifying additional factors which significantly influence production (e.g. account for regional differences within the PNW).

3.3	Guideline format

The guideline itself is composed of a series of connecting (linked) worksheets (spreadsheets) which are programmed using simple formula and linkages to adjust automatically when manual base data adjustments are made (hence the need to store an original master copy as a reference).

- File structure

Within the main directory are the following general files:

Main directory: Yardsyst, including:

Common.xls	- details common costs and assumptions (Common Factors 					spreadsheet)

Definit.doc 	- details of definitions and criteria for cost/production items

The set of spreadsheets are divided according to the yarding system, as follows:

Cable (cable yarding);

- 40’tower

40’tower, stroke delimber (files with name ending _40D)

40’tower, manual delimbing (files with name ending _40M)

- 50’swyar, 50’ swing yarder (files with name ending _50S)

- 70’tower

70’tower, stroke delimber (files with name ending _70D)

70’tower, manual delimbing (files with name ending _70M)

CTL; cut-to-length system (files with name ending _CTL)

Helicop (helicopter yarding);

- Hel_Ma, non pre-bunching (files with name ending _HMA)

- Hel_Pb, pre-bunching with harvester (files with name ending _HPB)

Shovel; shovel logging (files with name ending _SHV)

Tireskid; wheel skidder

FB_tirsk: wheel skidder, feller buncher (files with name ending _FWS)

MA_tirsk: wheel skidder, manual falling/delimbing (files with name ending 				_MWS)

Trkskid; track type skidder

FB_trksk: track skidder, feller buncher (files with name ending _FTS)

MA_trksk: track skidder, manual falling/delimbing (files with name ending 				_MTS)

- Worksheet structure

There are five individual spreadsheets for each yarding alternative analyzed, as follows:

a) Common Factors

All common costs and assumptions, i.e. those factors common to more than one machine (e.g. interest rate, wages, insurance and maintenance criteria).  Manual adjustments within this file will automatically recalculate all other relevant linked files.

b) Mach_file ending

This file contains one spreadsheet per machine and describes the calculation for the total cost (ownership and operating) for each individual machine assumed as a common component of that particular yarding system.

Costs have been calculated for selected models only within each machine type.  Each spreadsheet contains additional columns where additional models may be added to increase the sample size or where the guideline user wishes to include a specific machine model.

Capital costs are based on the respective dealer’s list price only.  This cost does not account for specific differences in machine quality, design and productivity.

c) Site_file ending

This shows the summary of the individual machine costs for that yarding system plus the auxiliary costs, resulting in a total US$/month figure for that operation (1 “side”).

A variation of +/- 10% is also automatically calculated to allow for additional influences on production not identified within this process (if required). In such case, criteria for the adoption of either the lower or the higher logging cost estimate should be identified and stated by the user.

d) Prod_mts

This spreadsheet contains the range of estimated production levels for the yarding system given the assumed forest and site variables.  These estimates were established through a survey of selected logging industry.

e) Lgct_file ending

This file displays the calculated range of estimated logging costs in both table and chart format, allowing an estimate of the cost of the logging operation in US$/Mbf for a given production level.

It is the responsibility of the user to select the appropriate production level and therefore estimate the logging cost for each site.  Extensive field experience and a working knowledge of logging production is recommended to achieve the most representative estimate of the likely logging cost for a given site

- Instructions

Step 1.		Install program diskette and enter main file

Step 2.		Select the appropriate yarding system from within the main file

Step 3.		Check assumptions within Common Factors spreadsheet (use default data 			unless adjustment is required to suit specific conditions).

Step 4.		Check assumptions within Machine Costs spreadsheet (use default data 			unless adjustment is required to suit specific conditions).

Step 5.		Check assumptions within Common Factors spreadsheet (use default data 			unless adjustment is required to suit specific conditions).

Step 6.		Check assumptions within Total Site Cost spreadsheet (use default data 			unless adjustment is required to suit specific conditions).

- Ensure equipment cost matches the total cost stated within each 				Machine Cost spreadsheet.

- Adjust auxiliary costs if necessary within Total Site Cost 					spreadsheet.

Step 7.		Estimate average daily production in truck loads (select appropriate truck 			load size).

Step 8.		Use the Logging Cost spreadsheet or the Range of Logging Cost chart to 			estimate the typical logging cost (in US$/MBF on truck) for the selected 			level of production.

Note:	Field appraisers may simply use the Range of Logging Cost chart  				for the appropriate yarding system to estimate an average logging 				cost during field appraisal, having been previously familiarized with 				the above workings of the guideline.

The results were not presented in $/cubic units due to the variable conversion factors for different log sizes.

4.	Machine description

The following table describes the individual machines used as a basis for estimating total site costs and production ranges.  Machine types, set-up configuration and accessories reflect common industry practice in the PNW.

| Machine Type | Description |
| --- | --- |
| Shovel Logger | Excavator base with raised cab and heel logging boom |
| Wheel Skidder | Rubber tire skidder with grapple. |
| Track Skidder | Track skidder with grapple. |
| 40’ Tower Yarder | Approx. 40’ trailer mounted, 4 drum drum (including strawline) tower yarder with skyline (slackline) and small motorized carriage/chokers. Capable of full lift and lateral yarding. |
| 50’ Swing Yarder | Approx. 50’ track mounted, full interlocking 4 drum (including strawline) swing yarder with running skyline and mechanical slack pulling carriage/chokers. Capable of full lift and lateral yarding. |
| 70’ Tower Yarder | Approx. 70’ track mounted, 5 drum (including strawline) tower yarder with skyline (slackline) and large motorized carriage/chokers. Capable of full lift and lateral yarding. |
| Harvester | Track or tire harvester with harvesting head. Capable of felling, delimbing and bucking. |
| Forwarder | Tire forwarder with grapple log loader. |
| Feller Buncher | Track feller buncher with felling head. |
| Helicopter | Approx. 11,500 certified pound lift capacity (assumed as equivalent to twin rotor Vertol 107-II). |
| Log Delimber | Stroke delimber on excavator base. |
| Log Loader | Excavator base with raised cab and heel logging boom. |

Note:	Although other machine types are used by some logging contractors, the above 	machine types and configurations are those most commonly used for logging in the 	PNW.  Where other machines are used and the cost structure/production range 	cannot be equated to any of the above machines, specific adjustments are required 	by the user for accurate logging cost estimates.

5.	Definitions and assumptions

5.1	Time availability

Scheduled days per year

Available calendar days per year; total of 365 days, less Saturdays, Sundays and public holidays; i.e. 365 total available days - (2x52 Saturdays and Sundays) - (6 holidays) = 255 available calendar days per year.

Operational availability

Scheduled days less interruptions due to bad weather, fire restrictions, site set up and equipment mobilization;

| Description of lost days | Shovel/skidder logging systems | Cable/CTL logging systems |
| --- | --- | --- |
| Short term bad weather (up to 3 days) | Compensated during week ends | Compensated during week ends |
| Longer term bad weather and agency restrictions - not able to compensate | 53 | 13 |
| Moving equipment (one day every 2 months) | 6 | 6 |
| Between jobs - lost days | 10 | 10 |
| Fire restrictions | 6 | 6 |
| Total | 75 | 35 |

Net (paid) hours/day

Assumed to be 8.5 hours/day for all logging systems except for CTL (9.0 hours/day) and helicopter (8.0 hours/day).

As average net hours/day for helicopter yarding can vary significantly, manual adjustments can be made within the Total Site Cost worksheet of the Helicopter Yarding Guide.

Mechanical availability

This make allowance for machine breakdown (days when the operator is present and working conditions are suitable but machine breakdown results in operational downtime). This factor is higher for more reliable “static” equipment (e.g. cable yarders) and lower for “mobile” in-forest, heavily worked equipment (e.g. skidders).

Month

Considered to be 1/12 of the year. It does not exactly represent the costs of any particular month, but represents an average for all months, over the full year.

5.2	Cost categories

Common costs

Costs common to all or several machines (e.g. fuel price, insurance rates, interest rates, wages). Includes all costs which can follow a common assumption and thus stated as a common cost. This allows a more accurate comparison of the cost of running different machines and the generation of hypothetical “what if?” scenarios.  It also facilitates more efficient update of the base data within the guide.

Machine costs

All costs associated with a specific machine, including:

Ownership costs: costs of owning the machine and usually not related to the level of activity (e.g. capital cost, depreciation, insurance); and

Operational costs: costs variable according to the level of production (e.g. fuel, tires, repairs).

Auxiliary (overhead) costs

Costs for operating a logging site which can not be associated with an individual machine (e.g. administration, supervision, fire truck, office support, crew trailer, etc.) but are essential in any logging operation.

Total site cost

Sum of individual machine cost and auxiliary costs. Represents the total cost of running the logging operation.

5.3	Cost items

Ownership costs

| Type of Cost | Definition/description | Source |
| --- | --- | --- |
| Purchase Price | Delivered new equipment with most common options/alterations. Whether the machine is new or used is irrelevant in this case as the monthly cost is based on an average between acquisition and resale throughout the ownership period of the machine. | Price list from regional dealers |
| Average Annual Investment | Represents an average of the investment throughout the period of ownership. Calculated as follows: (N+1)/(2N) x Delivered Price, being N the years of ownership. This is done to level out the capital cost independently of it being the first or the last year of the ownership period. | Calculated |
| Insurance | Insurance rate for the equipment stated as a yearly % of the Average Annual Investment. Assumed to be: 3% (in-woods equipment), 2% (on-landing equipment) per year. | Contractors |
| Personal property tax | Normally stated as a % depreciated value of each machine asset. Variable by county/state and so not included in costings. Provision to add manually within the Common Factors worksheet. | Contractors |
| Residual Value | Assumed to be trade-in value at replacement. Variable by machine type and function. | Dealers Contractors |
| Depreciation | Straight line depreciation over the ownership period. Represents the machine self pay-back distributed evenly during the working life. | Calculated |
| Interest over capital | Cost of immobilizing the capital. This is not the profit of the operation but the opportunity cost of the capital investment; i.e. the potential yield of the capital if it were not immobilized in logging equipment. | Calculated |

Operating costs

| Type of Cost | Definition/description | Source |
| --- | --- | --- |
| Repair Reserve | Represents an average cost of repairs during the ownership period of the equipment (including parts and labor). Repair costs generally increase with machine age, however the repair reserve assumes that the total repair cost is spread equally over the ownership period. The repair reserve is proportional both to the investment and to the working hours per year. | Dealers Literature Contractors |
| Fuel | Estimated using the “Cost Guide for Empirical Appraisals”- USDA (Revision 12, November 1983 guidelines). Fuel consumption is estimated based on horsepower (flywheel) rating (multiplied by 65% to adjust to intermittent use at median RPM) and checked with dealer estimates for verification. Fuel Consumption in gallons per hour = 0.05137 x Rated flywheel horsepower. (^1.01265) | USDA Flywheel horsepower (dealers) |
| Lub, filters and hydraulic | Expressed as a % of fuel cost per month. Assumed that 15% of the fuel cost (including labor cost). Recommended by USDA (above reference) at 7% fuel cost (excluding filter and labor). | USDA Dealers Contractor |
| Tires | Tires cost is considered for wheel skidders and forwarders due to relatively intensive tire wear for these machines. The cost of new tires are depreciated uniformly during the expected working life. | Dealers |
| Under-carriage (track) | Track cost is considered for track skidders and shovel loggers. The maintenance cost for the undercarriage is estimated separately, following the guidelines described in the “Caterpillar Performance Handbook”, Edition 26, page 17-30 (e.g. for track skidders the Basic Factor = 5.0, using moderate conditions of impact and abrasives ( multipliers I=0.2, A=0.2, Z=0.5)) | Dealers |
| Lines | For cable logging, the line (wire rope/cable) replacement is considered as a separate item, warranting a specific replacement reserve (US$/operating hour). | Dealers Contractors |
| Operator wage | Represents the total cost to the employer, not the net received by the employee; i.e. all inclusive (hiring and training costs, wages, overheads, etc.) and spread over the average employment period. | Oregon Assoc. Loggers |

Auxiliary (overhead) costs

Individual auxiliary costs are detailed within the Total Site Cost worksheet for each yarding system analyzed (except for helicopter yarding where the overheads are included in the hourly cost for the helicopter).

The following criteria were adopted for the reserve machinery and administrative components of the auxiliary cost:

- Reserve machinery

It is common for many logging operations to include a reserve machine (usually an older D5/D6 size crawler tractor) which operates relatively few hours per month mostly as a reserve machine and in site preparation/restoration.  This machine costs less to own and operate than a regular machine.

| Yarding system | Auxiliary Machine |
| --- | --- |
| 40’tower | Yes |
| 50’swing yarder | Yes |
| 70’tower | Yes |
| Shovel | No |
| CTL | Yes |
| Tire Skidder | Yes |
| Track Skidder | No |

Note:	Such a reserve machine is less commonly associated with track skidder and shovel 	yarding systems and was thus not included in the cost analysis for these systems.

- Administrative costs

Administrative costs associated with running a small office include: legal/accountancy, phone/fax., electricity, staff, rent etc.

It was assumed that on average, each logging operation consists of two “sides”, thus  the cost to each side is a proportion of the total calculated administrative cost, depending on size and yarding system.  The following table outlines the proportion of the total calculated administrative cost allocated to each side:

| Yarding system | Single side’s share of total calc. adm. cost |
| --- | --- |
| 40’tower | 33% |
| 50’swing yarder | 50% |
| 70’tower | 50% |
| Shovel | 33% |
| CTL | 50% |
| Tire Skidder | 50% |
| Track Skidder | 50% |

5.4	Ownership period, residual value and repair reserve - criteria

Ownership period is estimated taking into consideration the residual value and the repair reserve. The period should reflect the actual useful life of the machine and not necessarily the tax related depreciation period.  Longer ownership periods generally result in trade-offs such as reduced residual value and increased repair reserve.  The ownership period in years is a factor of the expected ownership period and will vary accordingly.

For most machines, the assumed period is approximately 7,000 - 12,000 hrs.  The larger (50 - 70’) self mounted cable yarders have considerably longer ownership periods (18,000 hrs.) compared to most smaller (40’) trailer mounted cable yarders.

Residual value is estimated taking into consideration the machine function, level of technology (likelihood of being made obsolete), and level of commonness.  It reflects the trade-in value and not the book value.

Repair reserve is proportional to both the investment and the working hour per year. It is stated primarily as % of a new machine (NM) per 1,000 hours of operation (%NM 1,000 hr), providing an estimate of the % of the investment for each 1,000 hrs. of work.

It can be also stated as % of the investment over the life span (ownership period) (%NM Life Span), i.e. the total of the repair reserve accumulated over the ownership period.  Usually machines require between 1/3 to 1/2 of the original investment in maintenance. The value is then converted to US$/hour.

5.5	Production units

The basic unit for production is a truck load, assumed to be 55,000 lb. by weight but variable according to log size (volume).  Estimated for each assumed age class as follows:

- 4,800 Mbf for mature logs (100 yrs.)

- 4,000 Mbf for private forest (45 yrs.)

- 3,700 for thinnings (40 yrs.)

- 3,200 for thinnings (30 yrs.)			(assuming 16’ short log length scale)

Where:	- One cubic meter (m3) is equivalent to 35.31 cubic feet.

- One cubic foot is equivalent  to 3.5 Bft for thinnings and 5.5 Bft for mature 	timber.

- One m3 is approximately equivalent to 124 Bft for thinnings and 194 Bft for 	mature wood (Loren Kellogg, OSU).

Note:

- Production estimates are based on the production of the “side” as a whole, rather than on individual machine production.  The critical component of the side’s production is assumed to be the yarding operation and, under normal circumstances, falling, delimbing/processing and loading do not restrict production levels (i.e. at least match the production of the yarding process).

- It is assumed that the crew is adequately trained, the machines perform to the assumed level of uptime (see Common Factors worksheet) and the weather is regular for the season.

5.6	Tree size

Assumed as follows:

| Size class | Volume/tree | DBH | Typical Age | Commercial Height (approx.) |
| --- | --- | --- | --- | --- |
| Thinning * | 70 Bft | 9” | 30 yrs. | 16’* 2 logs = 32’ |
| Thinning | 130 Bft | 12” | 40 yrs. | 16’* 3 logs = 48’ |
| Mature (private)* | 250 Bft | 18” | 45 yrs. | 16’* 4 logs = 64’ |
| Mature | 900 Bft | 28” | 100 yrs. | 16’* 7 logs = 112’ |

Note:	* tree sizes relevant to selected yarding systems only (i.e. not modeled for all 	yarding systems analyzed).

5.7	Harvest prescription

Assumed as follows:

| Prescription | Typical Age | Stand volume removed | Canopy removal | Comment |
| --- | --- | --- | --- | --- |
| Thinning * | 30 yrs. | 4,500 Bft/ac | 30% | Selective harvest |
| Thinning | 40 yrs. | 7,000 Bft/ac | 30% | Selective harvest |
| Regen cut (private)* | 45 yrs. | 35,000 Bft/ac | 90-95% | Typical “industrial clear-cut” |
| Partial cut | 100 yrs. | 20,000 Bft/ac | 50% | Retain 15-18 trees/ac |
| Regen cut | 100 yrs. | 50,000 Bft/ac | 70-90% | Retain <6 trees/ac |

Note:	* prescriptions relevant to selected yarding systems only (i.e. not modeled for all 	yarding systems analyzed).

5.8	Yarding distance

Assumed as follows:

| Yarding distance category | Distance (1 way) | Justification |
| --- | --- | --- |
| Short | 400 ft | Common shovel logging distance |
| Medium | 800 ft | Common PNW yarding distance |
| Long | 1,500 ft | Typical cable yarding productivity threshold |

5.9	Terrain class

Assumed as follows:

| Slope class | Grade | Criteria | Aerial yarding systems (cable, helicopter) | Ground yarding systems (tire) |  | Ground yarding systems (track) |
| --- | --- | --- | --- | --- | --- | --- |
| Flat | 0-30% | Normal operational limit (tire machines) | Uncommon* | Yes |  | Yes |
| Medium | 30-50% | Normal operational limit (track machines) | Yes |  | No | Yes |
| Steep | 50%+ | Traditional aerial yarding terrain | Yes |  | No | No |

* aerial yarding may be used on flat terrain where certain environmental and other 		planning constraints need to be met.

5.10	Wages and benefits

Assumed wage classes include:

| Description of the activity | Wage (net US$/hr) | Example |
| --- | --- | --- |
| Predominantly manual labor | 12.00 | Choker setter, chaser |
| Predominantly machine operation | 15.00 | Operator: feller buncher, tower |

Note:	The net (or take-home) wage includes typical over-time costs.

Overheads and benefits considered above the net hourly wage include the following:

| Item | % |
| --- | --- |
| Workers Compensation Insurance | 28.0 |
| Social Security Tax | 7.5 |
| State Unemployment Tax | 3.0 |
| Federal Unemployment Tax | 1.0 |
| Health & Life Insurance | 2.0 |
| Vacation | 1.0 |
| Others: personal protection equip., hiring costs, training, etc. | 5.0 |
| Total | 47.5 |

Note: 	Workers Compensation rate is based on an estimated average.  Actual rates will 	vary between companies depending on operation size, level of mechanization and 	safety performance record.

5.11	Manual Falling

The number of fallers required for each yarding system is a function of the operation performed.

For the purposes of the analysis and reflecting common practice, the following number of fallers per side were assumed:

| Type of operation | Logging System |  |
| --- | --- | --- |
|  | Track/tire skidder 50’swing yarder 70’tower | 40’tower Shovel logging |
| Manual Falling | 3.5 fallers | 3.0 fallers |
| Manual Falling and Delimbing | 2.0 fallers | 1.5 fallers |

Note:	Falling was assumed (and is commonly) to be carried out on a sub-contract basis.

5.12	Cable harvesting accessories

The carriage and lines of a cable system are significant cost items.  Depreciation and maintenance/repair of the carriage and lines are considered as separate cost items, according to the following cost schedules:

Carriage:

| Cable yarding system | Carriage type | Carriage invest. US$ | Carriage maintenance US$/hour | Carriage depreciation US$/hour | Rigging & Chokers US$ |
| --- | --- | --- | --- | --- | --- |
| 40’tower | Motor. | 34,000 | 5.00 | 3.96 | 15,000 |
| 50’ SY | Mech. | 15,000 | 1.00 | 1.80 | 25,000 |
| 70’tower | Motor. | 100,000 | 5.00 | 12.00 | 25,000 |

Lines:

| Cable yarding system | Repair reserve for lines US$/h of operation |
| --- | --- |
| 40’tower | 5.00 |
| 50’swing yarder | 8.00 |
| 70’tower | 8.00 |

6.	Acknowledgments

The World Forest Institute wishes to thank the many people within various sectors of the forest industry who contributed their time and knowledge to the compilation of this guide to estimating logging costs.  In particular, we wish to thank those loggers and logging equipment dealer representatives who provided invaluable advice and assistance.

************************************************************************