|  | Problem Set No.2: Horizontal Curve Geometry DUE 1/19/00 | Low-Volume Road Design FE 346 Winter Quarter |
| --- | --- | --- |

- Problem 1

Given DELTA = 43°05' and E = 172 ft (approx).  What is the degree

of curve?

- Problem 2

If DELTA = 30°12' and D = '2°48', find T and L.

- Problem 3

Using the data of Prob. 2 and assuming the PI to be at Sta.

42 + 16.6, what is the correct number for the PT?

- Problem 4

If the radius of a curve is 4,000 ft, calculate the deflection angle

for a 40-ft arc

- Problem 5

What is the middle ordinate M  for an arc of 100 ft, which sub-

tends a central angle of 8°00' ?

- Problem 6

Given D = 6°00' (arc definition) and the PC at Sta. 21 + 00.

With reference to the PC and the initial tangent, what are the coordinates x and y locating Sta. 23 + 00?

(also see Hickerson p. 67)

- Problem 7

Given DELTA = 22°36' and D = 4°. With the PC at Sta. 40 + 22.4,

calculate the deflection angles for the circular curve and check by

seeing that the last deflection (to the PT) = DELTA/2.

(see Hickerson p. 83)

- Problem 8

Given D = 1°15' and PC at Sta. 32, what is the "deflection distance" q for setting Sta. 33+00 ?

(see Hickerson p. 96)

- Problem 9

Given D = 1°15' and the PC at Sta. 32 + 20, what is the

"deflection distance" for setting Sta. 33+00 ?

(see Hickerson p. 96)

- Problem 10

Given DELTA = 47°12', PI = Sta. 58 + 22. Find T, E, and L for

D = 6°00' (arc definition); also stationing of the PC and PT.

- Problem 11

Same as Prob. 10, except for chord definition.

- Problem 12

Given DELTA = 24°20', R = 5,000 ft. Locate the PC and PT, if

PI = Sta. 36 + 42.32.

- Problem 14

In the layout of a 15° curve, if two equal chords are used instead of one 100-ft chord, what should be their length?

- Problem 15

In the layout of a 15° curve, if two equal chords are used to

establish a I00-ft arc, what should be their length?