|  | Curve Lay - Out By Deflection Angle | Low-Volume Road Design FE 346 Winter Quarter |
| --- | --- | --- |

| CURVE RADIUS |  | DEFL ANGLE/ FOOT |  | DEFLECTION ANGLES FOR THREE CORD LENGTHS |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | DEGREES |  |  |  |
|  |  |  | 20 FT Cord |  | 25 FT Cord | 30 FT Cord |  |
| 40 | 42.9718 |  | 14o |  | 18o | 20o |  |
| 50 | 34.3775 |  | 11o |  | 140 | 17o |  |
| 60 | 28.6478 |  | 10 |  | 12 | 14 |  |
| 70 | 24.5553 |  | 8 |  | 10 | 12 |  |
| 80 | 21.4859 |  | 7 |  | 9 | 11 |  |
| 90 | 19.0986 |  | 6 |  | 8 | 10 |  |
| 100 | 17.1887 |  | 6o |  | 7o | 9o |  |

EXAMPLE FOR RUNNING 50 FT RADIUS CURVE USING A 25 FT CORD

Note:  The first deflection angle ONLY is 140,  all others are double at 280.  This is due to the fact that the 140 defl. angle is turned from a line tangent to the curve ( PC ).