Exhibit 1

Field Location Techniques

Work Item or

Condition                                Techniques

_____________________________________________________________

1.  Overall     - Locate road using gradeline control

location          with Abney or clinometer when cross

slopes are greater than 15 percent.

- Locate road using horizontal control

with hand compass when cross slopes

are less than or equal to 15 percent

or in rolling hummocky ground with no

specific side slope.

- Fit road to the topography, minimizing

landform modifications.

- Run lines from specific control points

to general points.

- Tie into previously selected control

points using an altimeter.  Check

against profiles and maps.

- Exhaust other feasible alignment options

before locating road on exposed slopes.

________________________________________________________________

2.  Running         - The final design line will often be

steeper than the gradeline  located line.

The shorter the "shots" and the more broken

the terrain, the greater the  deviation.

This deviation is the least evident where

the located line is essentially the final

design line.

_____________________________________________________________

Located                Grade          Design Grade

2-4%       Approx. 1/2% steeper

5-10%      Approx. 1% steeper

10-14%      Approx. 2% steeper

- Grades should be slackened in broken terrain

to allow sufficient design flexibility.

- Flatten grade at potential drainage dip

locations.

- Grades of 0-2% should be avoided where

possible.

- Where the algebraic difference in grades

exceeds 5 percent, transition grade changes in

increments of 5 percent or less.  Separate

incremental transition by 25' to 50'.

________________________________________________________________

3.  Log landings    - Tractor landings should be located in

and logging       areas with gentle back slopes and low

systems           cut banks.

- Provide opportunities for off-road landing

access.

- Roads leaving landings should be located

with a grade of 0-5 percent for approximately

150'.

- For road segments with, or to be used as,

continuous landings, grades of 6 percent or

less are preferred.

- To the extent possible, locate roads on

topographic breaks to fit logging system

needs.

Work Item or

Condition                                Techniques

_____________________________________________________________

- Coordinate landing locations withturnout

opportunities.

- Reduce or minimize visual impacts by

avoiding noses of ridges for landings, if

possible.

4.  Switchbacks      - Locations should be limited to areas with

side slopes of 35 percent or less.

- Locate switchbacks to minimize visual

impacts using vegetation or topography  to

screen impacts.

- Mark approach grades and grades within

switchbacks in accordance with design

standards (4.32).

- Design grades will be steeper than located

grades on switchbacks.

- Lay out short radius switchbacks as circular

curves using radius point and cloth tape or by

deflection angle (2.33).

___________________________________________________________________

5.  Intersections   - Locations should be limited to areas

with side slopes of 25 percent or less.

- Intersection angles should not be less than

60 degrees (4.33).

- Grade of through road at intersections

should not exceed 4 percent.

- Grade of approach road should not exceed 6

percent (4.33).

- Grade of both roads should be the same until

adequate horizontal separation is achieved.

Work Item or

Condition                                Techniques

_____________________________________________________________

6.  Horizontal      - Approximate curve locations by establishing

curves            a series of points on curves (POCs).  Incre-

mental deflection angles should be less than

45 degrees.

- Lay out short radius curve as circular

curves using a radius point and cloth tape or

by deflection angle (2.33).

- Low standard roads may not require layout of

circular curves.

- Lay out curves so that sharp crest vertical

curves do not coincide with horizontal curves.

- Lay out alignment along natural curvature of

terrain to minimize cuts and fills and to

provide self-balanced sections.

- Check curves to see that they are  smooth

and without sharp breaks.  Check local

topography to be sure necessary curve widening

and turnouts can be accommodated.

________________________________________________________________

7.  Ridges (sharp   - Avoid abrupt vertical curves at ridge points.

and narrow)

- Lay out short radius curves as circular

curves using a radius point and cloth tape or

by deflection angles.

- Reduce grade to provide sufficient design

flexibility.

Work Item or

Condition                                Techniques

_____________________________________________________________

8.  Drainages  - Avoid paralleling for prolonged length.

and draws

- Avoid locations that will result in filling

of natural stream channel.

- Avoid sag curves over drainages.  Locate

vertical points of intersection (VPIs) in

vicinity of transition between cuts and fills

(4.32).

- Where possible, slacken the location grade

to 0-1 percent approximately 100'-200' before

crossing draw and extend 100'-200' beyond draw

or reduce  favorable grades by 5 percent,

adverse grades by 2 percent (reduces the need

for shifting or braking on curves).

- Where possible, cross at right angle and

from top of bank to top of bank.  Avoid

cutting down banks, if possible.

9.  Surface    - Drainage dips should not be located on

drainage     desing grades over 10 percent (4.43).

- If surface drainage is to be controlled by

rolling grades, incorporate grade breaks in

the road location.

- Avoid locating cross drainage on curves and

at turnouts.

10. Full bench - The marked grade line will be above the

anticipated design grade line and shifted into

the side hill 1/2 road width compared to self-

balanced areas.  Provide for transitions into

and out of full bench sections.

Work Item or

Condition                                Techniques

__________________________________________________________________

11. Instrument  - Check Abneys, clinometers, and checks

compasses daily.  Check other supporting

equipment and materials frequently.

__________________________________________________________________

12. Bridge sites    - Desirable features:

o Naturally confined stream

o Good approach alignment

o No grade changes or curves on bridge

o Solid foundations for abutments

o Straight stream course

o Crossing is perpendicular to the stream

o Bridge not located at grade sag.

o Bridge is significant cost item.

Location of economical crossing

may controllocation of approaches.

o See FSH 7709.56b, Drainage Structures

Handbook.

___________________________________________________________________

13.  Road      - Locate intersections to facilitate

closures    effective road closures where required.

- Locate closure points in vicinity of areas

suitable for turnarounds.

- Locate closure points that tie into natural

barriers.

____________________________________________________________________

14.  Scenic    - Where specified in design criteria and road

vistas      management objectives, identify off-road parking

areas in the vicinity of scenic features.

- Provide safe sight distance for vehicles entering

or leaving identified parking areas.

2.33 - Field Location Techniques for Special Situations.  In some

areas it is necessary to mark the field location of sharp radius

curves through switchbacks, draws, and ridges as a circular curve

regardless of the actual design method that will be used.

1.  Marking Curves by Deflection Angle.  Vegetation,

topography, or other factors may preclude locating a curve by

using a tape held at the radius point and marking the arc of the

curve.  In these cases, marking the curve by deflection angle and

chord can be effective, as shown in exhibit 1.

2.  Switchbacks.  Switchbacks can be located in the following

manner (and as illustrated in exhibit 2) unless the radius is

relatively large.  (For switchbacks with a radius greater than

100 feet, consider using the deflection angle method.)

a.  Step 1.  Extend the upper and lower gradelines of the

approaching alignment until they intersect.

b.  Step 2.  Shift the intersecting point uphill.  The

steeper the side slope, the greater the shift.  This

becomes the radius point of the curve.  The shift results

in better balance of earthwork.

c.  Step 3.  Mark the curve by measuring radially from

the radius point.

d.  Step 4.  Connect the point of curvature (PC) and

point of tangent (PT) of the curve to the upper and lower

gradelines with a horizontal line that follows the

natural ground contour.

3.  Locating and Carrying a Grade Through a Narrow Ridge.  A

road can be located to cross through a narrow ridge by using the

procedures described below and illustrated in exhibit 3.

a.  Step 1.  Select a location for crossing the ridge.

Extend the approaching gradeline to the beginning of the

curve (Point A).

b.  Step 2.  Using the radius point or deflection angle

methods, establish the horizontal alignment through the

selected ridge point.  The midpoint on the curve (Point

B) will lie approximately at the crest of the ridge.

c.  Step 3.  Starting at Point A, use an Abney or

clinometer to mark equal vertical step(s) up the ridge on

the approach side, and down the ridge on the other side

at the desired grade (G), while following the horizontal

alignment.  The number of steps up must equal the number

of steps down.  The grade setting on the survey

instrument must be reversed before stepping down.

d.  Step 4.  Where the topography will permit proceeding

on grade with a smaller deflection angle than is required

by following the horizontal curve (Point C), continue

running a gradeline along the ridge.