Recon Report

Jason Teller

Jesse Saunders

Cathy Lander

Paul Carpenter

FE 345

4/13/00

Road Name:




2000-2070 Connector Road





UW Common Name:
Between Kirkland Pass and Canyon Loop Areas


Road Class:




Secondary Road

Initial Designers:
Jason Teller, Jesse Saunders, Cathy Lander, Paul Carpenter


Total Length (stations):


23 stations


Road Access Description:


2070 Road to 2000 Road


Takeoff Location:
T16N R04E NE ¼ of NW ¼ of Section 34




Termination Location:
T16N R04E NW ¼ of NW ¼ of Section 34



Quantity of Settings Accessed:

Unknown


Area of Sales Accessed (acres):

Unknown


Road Status: 




Grade lined, P-lined, traversed

Soils Classification:



Baumgard Soil Series


Stream Crossings Information:

None


Switchback/Curve Information:

4 to 5 slight curves forcasted


Rock Outcrop Information:


None

Comments:
Starting at T16N R04E NE ¼ of NW ¼ of Section 34, at the landing on the 2070 road, the group began numbering stations for the grade line.  The beginning station number was 0 + 00.  We ended the grade line at 23+00 on the 2000 road.  The initial bearing was S10 W and we entered the 2000 road at a bearing of S70 W.  20 year-old Douglas-fir was encountered from 0 + 00 to 10 + 00.  Salal was the major understory species in this area.  Also, due to the densely planted Douglas-fir, 25 foot stations were used.  From the 10 + 00 station to 23 + 00, the grade line was run through a 5 year-old clearcut.  

Feature
Station Number(s)

Skid Road followed
~ 7 + 00 to 8 + 00

20 year-old Douglas-fir
0 + 00 to 10 + 00

5 year-old clearcut
10 + 00 to 23 + 00

Steepest Side Slope (46 %)
9 + 00

End Station Number
23 + 00

