
Washington State Department of Natural Resources


Geographic Information System


April 1997

DETAILED LAYER DESCRIPTION
LAYER: POCAL

GEN.SOURCE: POCA/LULC/ACTIVITY combined data

CODE DOCUMENT: None

CONTACT: Rick Tavenner (FRD)               PHONE: 360‑902‑1368

COVER TYPE:  Spatial polygon coverage

DATA TYPE:  Data derived from other covers

DESCRIPTION:
The POCAL data layer combines the spatial delineations and  attributes from the POCA layer (Public land survey, upland Ownership managed by DNR, County boundaries, and DNR Administrative boundaries) with DNR resource management  activities and land use/cover data.  The POCAL layer  generally retains POCA data for lands managed by DNR, i.e. the same coverage as LULC.

    
 The land use/cover and management activity data themes are  documented separately as LULC and ACTIVITY data sets.  POCAL attribute data are stored in polygon attribute tables  (.pat) and INFO expansion files.

COORDINATE SYSTEM: 
WA State Plane South Zone (5626)   (N. zone converted to S. zone)

COORDINATE UNITS:  
Feet

HORIZONTAL DATUM: 
NAD27

PROJECTION NAME:   
Lambert Conformal Conic

FILE DESCRIPTIONS BY LAYER
 LAYER        FILE             FILE DESCRIPTION

 NAME         NAME
 POCAL        POCAL.PAT        POCAL polygon attribute file

ITEM DESCRIPTIONS BY FILE & LAYER‑‑POCAL
 FILE

ITEM

ITEM

 NAME             NAME
DESCRIPTION
 POCAL.PAT    AREA

Program assigned polygon area in cover units

                  
PERIMETER
Length of polygon perimeter

                  
POCAL#           Program assigned spatial feature identifier

                  
POCAL‑ID
User‑controlled spatial feature identifier

                 
 LULC.NO
Unique statewide LULC polygon number

                  
STAND.NO
Unique statewide stand number

                  
TOWN.ID
Township/Range & Dir.from Willamette Meridian

                  
SECTION.ID
Public land survey section identifier

                  
STATE
Code ID'ing Canadian/U.S./Wash. State lands

                  
COUNTY
State county code identifier

                  
RESERV           Code ID'ing type of reserve or disputed land

                  
PARK

National Park ID (not maintained statewide)

                 
 LAND.MARINE.FLG  Uplands (1) or marine waters (2) flag

                  
BLK.PLN.AREA
Code identifying DNR Block Planning Area

                  
ADMIN.CODE
DNR administrative unit code

                  
ADMIN.TYPE
Type of land parcel administration‑‑curr.not used

                  
COV_NUM
Used to ID coverages in "appends" & "splits"

                  
POLY.NUM
DNR management parcel no.‑‑unique statewide

                  
SUR.OWN
Surface trust land ownership code

                  
CNT.TRS
Trust Code of lands being sold under contract

                  
TIM.OWN
Timber ownership code

                  
MIN.OWN
Code for trust having mineral rights

                  
TIM.MO
Timber removal deadline mo.on land exchange parcel

                  
TIM.YR
Timber removal deadline yr.on land exchange parcel

                  
TIM.WHO
Non‑DNR party involved in timbered land exchange

                  
ACRES
Legal acres of state ownership

                  
FLAG

Warning to check tract book for details

                  
DATE

See Detailed Item Description for definition

                  
UNSYM
Unique poly id: (TOWN.ID * 10000) + cover‑ID

                  
POCAL.NO
Unique statewide POCAL polygon number

                  
REGION
Code identifying DNR administrative region

                  
DISTRICT
Code identifying DNR administrative district

                  
TWP.CODE
Township code

                  
TWP.LULC.NO
Township‑unique inventory polygon identifer

                  
TWP.STAND.NO
Township‑unique stand identifer

                  
POCA.NO
Unique statewide POCA layer polygon number

ITEM FORMATS BY FILE & LAYER
 LAYER        FILE             
ORDER
ITEM               IN  
OUT 
FMT DEC

 NAME         NAME                 
 
NAME             WDTH WDTH TYP

 ------------------------------------------------------------------------------------------------------------------------

 POCAL        POCAL.PAT            
1 
AREA

4   
12 
F     3

                                   


2 
PERIMETER
4   
12 
F     3

                                   


3 
POCAL#
4    
5 
B     0

                                   


4 
POCAL‑ID
4    
5 
B     0

                                   


5 
LULC.NO
8    
8 
I     0

                                   


6 
STAND.NO
8    
8 
I     0

                                   


7 
TOWN.ID
7    
7
I     0

                                   


8 
SECTION.ID
5    
5 
I     0

                                   


9 
STATE
1    
1 
I     0

                                  


10 
COUNTY
2    
2 
I     0

                                  


11 
RESERV
3    
3 
I     0

                                  


12 
PARK                3    
3 
I     0

                                  


13 
LAND.MARINE.FLG     3    3
 I     0

                                  


14 
BLK.PLN.AREA 3    
3 
I     0

                                  


15 
ADMIN.CODE    4    
4 
I     0

                                  


16 
ADMIN.TYPE
3    
3 
I     0

                                  


17 
COV_NUM
2    
2 
I     0

                                  


18 
POLY.NUM
5    
5 
I     0

                                  


19 
SUR.OWN
2    
2 
I     0

                                  


20 
CNT.TRS
2    
2 
I     0

                                  


21 
TIM.OWN
2    
2 
I     0

                                  


22 
MIN.OWN
2    
2 
I     0

                                  


23 
TIM.MO
2    
2 
I     0

                                  


24 
TIM.YR
4    
4 
I     0

                                  


25 
TIM.WHO
7    
7 
C     0

                                  


26 
ACRES
7    
7 
N     2

                                  


27 
FLAG

1    
1 
I     0

                                  


28 
DATE

8   
10 
D     0

                                  


29 
UNSYM
12   
12 
I     0

                                  


30 
POCAL.NO
8    
8 
I     0

                                  


31 
REGION
2    
2 
I     0

                                  


32 
DISTRICT
3    
3 
I     0

                                  


33 
TWP.CODE
4    
4 
I     0

                                  


34 
TWP.LULC.NO  4    
4 
I     0

                                  


35 
TWP.STAND.NO 4    
4 
I     0

                                  


36 
POCA.NO
8    
8 
I     0


DETAILED ITEM DEFINITION
ITEM:   POCAL#

TITLE:  Program assigned spatial feature identifier

FORMAT: 
INPUT   OUTPUT   DATA   DECIMAL     ARRAY   ARRAY

                
WIDTH   WIDTH    TYPE   PLACES      OCCUR.  INDEX

                 
--------------------------------‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑

                    
 4      

 5       B       
 0          
0       0

CODE TABLE OR VALUE RANGE: ‑‑‑

DESCRIPTION:
This is a unique sequence number which is automatically generated by the ARC/INFO software.  This internal number is used by the ARC/INFO software as a direct  access record number for the feature.  Therefore the number should not be changed in any way by the user.  It is also used to describe topological relationships between coverage features.

ITEM:   POCAL‑ID

TITLE:  User‑controlled spatial feature identifier

FORMAT:
INPUT   OUTPUT   DATA   DECIMAL     ARRAY   ARRAY

WIDTH   WIDTH    TYPE   PLACES      OCCUR.  INDEX

--------------------------------‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑

4       

5       B       
 0          
0       0

CODE TABLE OR VALUE RANGE: ‑‑‑

DESCRIPTION:
This is a number assigned to each feature by the user.  It need not be unique and will be maintained by the system as the coverage is processed.  It can be used to represent an attribute for the feature or to relate additional attribute information to the feature.

DETAILED ITEM DEFINITION
ITEM:   POCAL.NO

TITLE:  Unique statewide POCAL polygon number

FORMAT: 
INPUT   OUTPUT   DATA   DECIMAL     ARRAY   ARRAY

WIDTH   WIDTH    TYPE   PLACES      OCCUR.  INDEX

-------------------------------‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑

8       

8       I       
 0         

 0       0

CODE TABLE OR VALUE RANGE: 10001‑21019999

DESCRIPTION:
Unique statewide POCAL polygon number.  In the POCAL layer there may be several polygons with the same POCA.NO as a result of stand or activity lines dividing the original POCA polygons.  Therefore, POCAL.NO is used to provide a unique polygon identifier in that system environment.

The polygon identifier is derived by multiplying the township code by 10,000 and then adding the number stored in POCAL-ID for that polygon, i.e. (TWP.CODE *10000) + POCAL-ID

ITEM:   TIM.OWN

TITLE:  Timber ownership code

FORMAT: 
INPUT   OUTPUT   DATA   DECIMAL     ARRAY   ARRAY

WIDTH   WIDTH    TYPE   PLACES      OCCUR.  INDEX

------------------------------------‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑

2       

2       I        
0          

0       0

CODE TABLE OR VALUE RANGE: OWN.CODE

DESCRIPTION:
If this field contains a zero (0), the trust owning the timber is actually the same as the trust with designated surface ownership (SUR.OWN).


CODE TABLES
CODE TABLE NAME:  SECTION.ID.CODE

 ‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑

 CODE
MAP/REPORT
MAP 
CODE DESCRIPTION

LABEL
SYMB
 ‑‑‑‑‑‑‑‑‑  ‑‑‑‑‑‑‑‑‑‑‑‑ ‑‑‑‑ ‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑
    101        1            
0 
Regular public land survey section ID (+100)

    102        2           
0 
Regular public land survey section ID (+100)

    103        3            
0 
Regular public land survey section ID (+100)

    104        4            
0 
Regular public land survey section ID (+100)

    105        5            
0 
Regular public land survey section ID (+100)

    106        6            
0 
Regular public land survey section ID (+100)

    107        7            
0 
Regular public land survey section ID (+100)

    108        8            
0 
Regular public land survey section ID (+100)

    109        9            
0 
Regular public land survey section ID (+100)

    110       10            
0 
Regular public land survey section ID (+100)

    111       11            
0 
Regular public land survey section ID (+100)

    112       12            
0 
Regular public land survey section ID (+100)

    113       13            
0 
Regular public land survey section ID (+100)

    114       14            
0 
Regular public land survey section ID (+100)

    115       15            
0 
Regular public land survey section ID (+100)

    116       16            
0 
Regular public land survey section ID (+100)

    117       17            
0 
Regular public land survey section ID (+100)

    118       18            
0 
Regular public land survey section ID (+100)

    119       19            
0 
Regular public land survey section ID (+100)

    120       20            
0 
Regular public land survey section ID (+100)

    121       21            
0 
Regular public land survey section ID (+100)

    122       22            
0 
Regular public land survey section ID (+100)

    123       23           
0 
Regular public land survey section ID (+100)

    124       24           
0 
Regular public land survey section ID (+100)

    125       25           
0 
Regular public land survey section ID (+100)

    126       26           
0 
Regular public land survey section ID (+100)

    127       27           
0 
Regular public land survey section ID (+100)

    128       28           
0 
Regular public land survey section ID (+100)

    129       29            
0 
Regular public land survey section ID (+100)

    130       30            
0 
Regular public land survey section ID (+100)

    131       31            
0 
Regular public land survey section ID (+100)

    132       32            
0 
Regular public land survey section ID (+100)

    133       33            
0 
Regular public land survey section ID (+100)

    134       34            
0 
Regular public land survey section ID (+100)

    135       35            
0 
Regular public land survey section ID (+100)

    136       36            
0 
Regular public land survey section ID (+100)

    137       37            
0 
Donation Land Claim (DLC) number (+100)

    138       38            
0 
Donation Land Claim (DLC) number (+100)


CODE TABLES
CODE TABLE NAME:  SECTION.ID.CODE

 ‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑

 CODE
MAP/REPORT
MAP 
CODE DESCRIPTION

LABEL
SYMB
 ‑‑‑‑‑‑‑‑‑  ‑‑‑‑‑‑‑‑‑‑‑‑ ‑‑‑‑ ‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑
    139       39            
0 
Donation Land Claim (DLC) number (+100)

    140       40            
0 
Donation Land Claim (DLC) number (+100)

    141       41            
0 
Donation Land Claim (DLC) number (+100)

    142       42            
0 
Donation Land Claim (DLC) number (+100)

    143       43            
0 
Donation Land Claim (DLC) number (+100)

    144       44            
0 
Donation Land Claim (DLC) number (+100)

    145       45            
0 
Donation Land Claim (DLC) number (+100)

    146       46            
0 
Donation Land Claim (DLC) number (+100)

    147       47            
0 
Donation Land Claim (DLC) number (+100)

    148       48            
0 
Donation Land Claim (DLC) number (+100)

    149       49            
0 
Donation Land Claim (DLC) number (+100)

    150       50            
0 
Donation Land Claim (DLC) number (+100)

    151       51            
0 
Donation Land Claim (DLC) number (+100)

    152       52            
0 
Donation Land Claim (DLC) number (+100)

    153       53            
0 
Donation Land Claim (DLC) number (+100)

    154       54            
0 
Donation Land Claim (DLC) number (+100)

    155       55            
0 
Donation Land Claim (DLC) number (+100)

    156       56            
0 
Donation Land Claim (DLC) number (+100)

    157       57            
0 
Donation Land Claim (DLC) number (+100)

    158       58            
0 
Donation Land Claim (DLC) number (+100)

    159       59            
0 
Donation Land Claim (DLC) number (+100)

    160       60            
0 
Donation Land Claim (DLC) number (+100)

    161       61            
0 
Donation Land Claim (DLC) number (+100)

    162       62            
0 
Donation Land Claim (DLC) number (+100)

    163       63            
0 
Donation Land Claim (DLC) number (+100)

    164       64            
0 
Donation Land Claim (DLC) number (+100)

    165       65            
0 
Donation Land Claim (DLC) number (+100)

    166       66            
0 
Donation Land Claim (DLC) number (+100)

    167       67            
0 
Donation Land Claim (DLC) number (+100)

    168       68            
0 
Donation Land Claim (DLC) number (+100)

    169       69            
0 
Donation Land Claim (DLC) number (+100)

    170       70            
0 
Donation Land Claim (DLC) number (+100)

    171       71            
0 
Donation Land Claim (DLC) number (+100)

    172       72            
0 
Donation Land Claim (DLC) number (+100)

    301      301            
0 
Unsurveyed area

    302      302            
0 
Unsurveyed area

    303      303            
0 
Unsurveyed area

    304      304            
0 
Unsurveyed area

    305      305            
0 
Unsurveyed area


CODE TABLES
  CODE TABLE NAME:  SECTION.ID.CODE

 ‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑

 CODE    MAP/REPORT   MAP  CODE DESCRIPTION

           
 LABEL        
SYMB
-------------------------------‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑
    306      306            
0 
Unsurveyed area

    307      307            
0 
Unsurveyed area

    308      308            
0 
Unsurveyed area

    309      309            
0 
Unsurveyed area

    310      310            
0 
Unsurveyed area

    311      311            
0 
Unsurveyed area

    312      312          
0 
Unsurveyed area

    396      296          
0 
Donation Land Claim (DLC) number (+100)

    418       18           
 0 
Indian Allotment (+400)

    419       19           
 0 
Indian Allotment (+400)

    420       20           
 0 
Indian Allotment (+400)

    421       21           
 0 
Indian Allotment (+400)

    422       22           
 0 
Indian Allotment (+400)

    423       23           
 0 
Indian Allotment (+400)

    424       24           
 0 
Indian Allotment (+400)

    425       25           
 0 
Indian Allotment (+400)

    426       26           
 0 
Indian Allotment (+400)

    427       27           
 0 
Indian Allotment (+400)

    428       28           
 0 
Indian Allotment (+400)

    433       33           
 0 
Indian Allotment (+400)

    435       35           
 0 
Indian Allotment (+400)

    436       36           
 0 
Indian Allotment (+400)

  CODE TABLE NAME:  STATE.CODE

 ‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑

 CODE
MAP/REPORT   MAP  CODE DESCRIPTION

LABEL
SYMB
----------------------------------‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑
  1         IN STATE
0
In state (Washington)

  5         IN USA
0 
Out of state but in USA

  9         NON‑US
0 
Out of USA

CODE TABLES
CODE TABLE NAME:  PARK.CODE

  ‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑

CODE       MAP/REPORT  
 MAP  CODE DESCRIPTION

LABEL

SYMB
----------------------------------‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑
    1       NAT PARK        
0 
NATIONAL PARK

    8       UNKNOWN PARK    
0 
UNRESOLVED PARK

  999
NON‑PARK        
0 
OUTSIDE PARK BOUNDARIES

CODE TABLE NAME:  COUNTY.CODE

 ‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑

CODE       MAP/REPORT  
 MAP CODE DESCRIPTION

            
LABEL        

SYMB
-----------------------------------‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑
   1        ADAMS           
0 
Adams County

   2        ASOTIN          

0 
Asotin County

   3        BENTON          
0 
Benton County

   4        CHELAN          
0 
Chelan County

   5        CLALLAM         
0 
Clallam County

   6        CLARK           

0 
Clark County

   7        COLUMBIA        
0 
Columbia County

   8        COWLITZ         
0 
Cowlitz County

   9        DOUGLAS         
0 
Douglas County

  10        FERRY           

0 
Ferry County

  11        FRANKLIN        
0 
Franklin County

  12        GARFIELD        
0 
Garfield County

  13        GRANT           
0 
Grant County

  14        GRAYS HARBOR    
0 
Grays Harbor County

  15        ISLAND          
0 
Island County

  16        JEFFERSON       
0 
Jefferson County

  17        KING            

0 
King County

  18        KITSAP          

0 
Kitsap County

  19        KITTITAS        
0 
Kittitas County

  20        KLICKITAT       
0 
Klickitat County

  21        LEWIS           

0 
Lewis County

  22        LINCOLN         
0 
Lincoln County

  23        MASON           
0 
Mason County

CODE TABLES
CODE TABLE NAME:  COUNTY.CODE

 ‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑

CODE       MAP/REPORT  
 MAP CODE DESCRIPTION

            
LABEL        

SYMB
-----------------------------------‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑
  24        OKANOGAN       
0 
Okanogan County

  25        PACIFIC         

0 
Pacific County

  26        PEND OREILLE    
0 
Pend Oreille County

  27        PIERCE          

0 
Pierce County

  28        SAN JUAN        
0 
San Juan County

  29        SKAGIT          
0 
Skagit County

  30        SKAMANIA        
0 
Skamania County

  31        SNOHOMISH       
0 
Snohomish County

  32        SPOKANE        
0 
Spokane County

  33        STEVENS         
0 
Stevens County

  34        THURSTON        
0 
Thurston County

  35        WAHKIAKUM       
0 
Wahkiakum County

  36        WALLA WALLA     
0 
Walla Walla County

  37        WHATCOM         
0 
Whatcom County

  38        WHITMAN         
0 
Whitman County

  39        YAKIMA          
0 
Yakima County

CODE TABLE NAME:  BLK.PLN.AREA.CODE

‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑

CODE       MAP/REPORT
MAP  
CODE DESCRIPTION

     LABEL        

SYMB
--------------------------------‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑
  105

ANDERSON

0
Anderson Block

  108

BEAR RIVER

0
Bear River Block

  110       
BLACK MT        
0 
Black Mountain Block

  120       
BOGACHIEL       
0 
Bogachiel Block

  130       
CAPITOL FOR     
0 
Capitol Forest Block

  140       
CAVANAUGH       
0 
Cavanaugh Block (from Pilchuck Cr. east)

  150       
CLALLAM         
0 
Clallam Block

  160

COLUMBIA RIV    
0 
Columbia River Block

  170       
COWICHE         
0 
Cowiche Block

  178

CRAWFORD

0
Crawford Block

  180       
CRESC‑ELWAH     
0 
Crescent‑Elwah Block


CODE TABLES
CODE TABLE NAME:  BLK.PLN.AREA.CODE

‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑

CODE      
 MAP/REPORT   
MAP  CODE DESCRIPTION

           

LABEL        

SYMB
---------------------------------------------------------------‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑

  190       
CULTUS MT       
0 
Cultus Mountain Block

  195

DEEP CREEK

0
Deep Creek Block

  200       
DOTY HILLS      
0 
Doty Hills Block

  210       
EAST NASELLE    
0 
East Naselle Block

  220       
E SHANNON       
0 
East Shannon Block

  230       
ELBE HILLS      
0 
Elbe Hills Block

  240       
ELOCHOMAN       
0 
Elochoman Block

  250       
GLENWOOD        
0 
Glenwood Block

  260       
GREEN MT        
0 
Green Mountain State Forest

  270       
GREEN RIVER     
0 
 River State Forest

  277

HARMONY

0
Harmony Block

  279

HARRIS ROAD
0
Harris Road Block

  280       
H.O. BLOCK      
0 
Harry Osbourn Block

  290       
HOH‑CLEARWTR    
0 
Hoh‑Clearwater Block

  300       
HOOD CANAL      
0 
Hood Canal State Forest

  310       
LAKE CR         
0 
Lake Creek Block

  320       
LEWIS RIVER     
0 
Lewis River Block

  327

LINCOLN CR

0
Lincoln Creek Block

  330       
L PEND OREIL    
0 
Little Pend Oreille Block

  340       
LOOMIS          
0 
Loomis Block

  350       
LOUP LOUP       
0 
Loup Loup Block

  360       
LOW CHEHALIS    
0 
Lower Chehalis Block

  370       
MARBLEMOUNT     
0 
Marblemount Block

  380       
MARCKWORTH      
0 
Marckworth State Forest

  385

MESKILL

0
Meskill Block

  386

MINERAL

0
Mineral Block

  388

MORTON

0
Morton Block

  390       
NARCISSE        
0 
Narcisse Block

  400       
NOOKSACK        
0 
Nooksack Block

  410       
P & E           

0 
P & E Block

  411

PACKWOOD

0
Packwood Block

  416

PLEASANT VLY
0
Pleasant Valley 

  420       
RADAR RIDGE     
0 
Radar Ridge Block

  422

RANDEL

0
Randel Block

  428

SALMON CR

0
Salmon Creek Block

  430       
SAUK            

0 
Sauk Block

  435       
SIOUXON         
0 
Siouxon Block

  440       
SKAMOKAWA       
0 
Skamokawa Block

  450       
SQUALLY JIM     
0 
Squally Jim Block


CODE TABLES
CODE TABLE NAME:  BLK.PLN.AREA.CODE

‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑

CODE      
 MAP/REPORT   
MAP  CODE DESCRIPTION

           

LABEL        

SYMB
---------------------------------------------------------------‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑

  455

SMITH RANCH
0
Smith Ranch Block

  460       
SULTAN/PILCH    
0 
Sultan/Pilchuck Block

  470       
SUMMAS MT       
0 
Summas Mountain Block

  480       
TAHOMA          
0 
Tahoma State Forest

  490       
TAHUYA          
0 
Tahuya State Forest

  500       
TIGER MT        
0 
Tiger Mountain State Forest

  510       
TOUTLE          
0 
Toutle Block

  520       
VAN ZANDT DK    
0 
Van Zandt Dike Block

  530       
W KLICKITAT     
0 
West Klickitat Block

  540       
WILLAPA HILL    
0
 Willapa Hills Block

  550       
WILLM/DAVIS     
0
 Williams/Davis Block

  560       
WINSTON CR      
0 
Winston Creek Block

  570       
YACOLT BURN     
0 
Yacolt Burn Block (East and West)

  999       
UNBLOCKED       
0 
No Block Designation

CODE TABLE NAME:  RESERV.CODE

‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑

CODE       MAP/REPORT  
MAP  
CODE DESCRIPTION

     LABEL        
SYMB

 ‑‑‑‑‑‑‑‑‑  ‑‑‑‑‑‑‑‑‑‑‑‑ ‑‑‑‑ ‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑
    1
INDIAN RESER    
0 
INDIAN RESERVATION

    2       MILITARY RES    
0 
MILITARY RESERVATION

    3       US‑CAN DISP     
0 
US‑CANADIAN DISPUTED LAND

    8       UNKNOWN         
0 
UNKNOWN RESERVATION

  999
NOT RESERVAT    
0 
NON‑RESERVATION LAND

CODE TABLES
CODE TABLE NAME:  FLAG.CODE

‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑

CODE       MAP/REPORT   
MAP  
CODE DESCRIPTION

     LABEL        
SYMB

 ‑‑‑‑‑‑‑‑‑  ‑‑‑‑‑‑‑‑‑‑‑‑ ‑‑‑‑ ‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑
  0         NONE            

0 
No question on ownership

  1         SUR.OWN         
0 
Question on surface ownership

  2         MIN.OWN         
0 
Question on mineral ownership

  3         TIM.OWN         
0 
Question on timber ownership

  4         COMBINATION     
0 
Question on more than one type of ownership


CODE TABLES
CODE TABLE NAME:  ADMIN.CODE.CODE

‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑

CODE      
MAP/REPORT
MAP  CODE DESCRIPTION

          

 LABEL       

SYMB
--------------------------------------------------------------------‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑

   100     
SOUTHEAST       
0 
Southeast Region

   110      
ALPINE

0 
Southeast         Alpine

   120      
KLICKITAT       
0 
Southeast         Klickitat

   130      
COLUMBIA BAS    
0 
Southeast         Columbia Basin

   131      
EPHRATA         
0 
Southeast         Columbia Basin   Ephrata

   134      
RITZVILLE       
0 
Southeast         Columbia Basin   Ritzville

   140      
SNAKE RIVER     
0 
Southeast         Snake River

   142      
TRI‑CITIES      
0 
Southeast         Snake River
Tri‑Cities

   143      
DAYTON          
0 
Southeast         Snake River
Dayton

   200      
OLYMPIC         
0 
Olympic Region

   210      
STRAITS         

0 
Olympic
Straits

   214      
PORT ANGELES    
0 
Olympic
Straits

Port Angeles

   215      
PRT TOWNSEND    
0 
Olympic
Straits

Port Townsend

   220      
OZETTE          

0 
Olympic
Ozette

   230

HOH


0
Olympic
Hoh

   250      
NO GRAYS HAR    
0 
Olympic
No.Grays Harbor

   251      
QUINAULT        
0 
Olympic
No.Grays Harbor  Quinault

   252      
HUMPTULIPS      
0 
Olympic
No.Grays Harbor  Humptulips

   400      
CENTRAL         
0 
Central Region

   410      
BLACK HILLS     
0 
Central

Black Hills District

   413

MIMA


0
Central

Black Hills
Mima

   420      
LEWIS           

0 
Central

Lewis

   424

WINSTON

0
Central

Lewis

Winston

   430      
PACIFIC         

0 
Central

Pacific

CODE TABLE NAME:  ADMIN.CODE.CODE

‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑

CODE      
MAP/REPORT
MAP  CODE DESCRIPTION

          

 LABEL       

SYMB
--------------------------------------------------------------------‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑

   431      
FRANCIS         
0 
Central

Pacific

Francis

   432      
NASELLE        
 0 
Central

Pacific

Naselle

   433      
PE ELL          

0 
Central

Pacific

Pe Ell

   434

RAYMOND

0
Central

Pacific

Raymond

   900      
SO PUGET        
0 
S.Puget Sound Region

   910      
SHORELINE       
0 
S.Puget Sound     Shoreline District

   913      
SEABECK         
0 
S.Puget Sound     Shoreline
Seabeck

   920      
HOOD CANAL      
0 
S.Puget Sound     Hood Canal

   921      
HOODSPORT       
0 
S.Puget Sound     Hood Canal       Hoodsport

   922      
BELFAIR         
0 
S.Puget Sound     Hood Canal       Belfair

   950      
RAINIER         

0 
S.Puget Sound     Rainier District

   952      
EATONVILLE      
0 
S.Puget Sound     Rainier          Eatonville

   953      
ENUMCLAW        
0 
S.Puget Sound     Rainier          Enumclaw

   960      
KING            

0 
S.Puget Sound     King District

   961      
DUVALL          
0 
S.Puget Sound     King             Duvall

   962      
ISSAQUAH        
0 
S.Puget Sound     King             Issaquah

   963      
MARITIME        
0 
S.Puget Sound     King             Maritime

  1100      
SOUTHWEST       
0 
Southwest Region

  1110

ST. HELENS

0
Southwest
St. Helens  

  1111

TOUTLE

0
Southwest
St. Helens
Toutle

  1112

GRAYS

0
Southwest
St. Helens
Grays

  1113      
KALAMA          
0 
Southwest         St.Helens
Kalama

  1120

YACOLT          
0 
Southwest        
Yacolt

  1121

COUGAR

0
Southwest
Yacolt

Cougar

  1122      
STEVENSON       
0 
Southwest
Yacolt

Stevenson

  1123      
BATTLEGROUND    
0 
Southwest
Yacolt

Battleground

  1900      
NORTHWEST       
0 
Northwest Region

  1910      
BAKER           

0 
Northwest     
Baker

   1911

DEMMING

0
Northwest
Baker

Demming

   1912

HAMILTON

0
Northwest
Baker

Hamilton

  1920      
CASCADE         
0 
Northwest        
Cascade

 1921

BRYANT

0
Northwest
Cascade
Bryant

 1922

DARRINGTON
0
Northwest
Cascade
Darrington

 1923

MONROE

0
Northwest
Cascade
Monroe

  1930      
SAN JUAN        
0 
Northwest        
San Juan

  2300      
NORTHEAST      
0 
Northeast Region

  2310      
HIGHLANDS       
0 
Northeast        
Highlands

  2320      
SO OKANOGAN     
0 
Northeast        
South Okanogan

  2330      
NO COLUMBIA     
0 
Northeast        
No.Columbia

  2340      
ARCADIA         
0 
Northeast        
Arcadia

  9999      
NONE            

0 
OUTSIDE ALL LOCAL BOUNDARIES

CODE TABLES
CODE TABLE NAME:  REGION.CODE

‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑

CODE       MAP/REPORT   
MAP  
CODE DESCRIPTION

LABEL        

SYMB

------------------------------‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑
   1        SE              

0 
SOUTHEAST

   2        OLY             

0 
OLYMPIC

   4        CEN             

0 
CENTRAL

   9        SPS             

0 
SOUTH PUGET SOUND

  11        SW              

0 
SOUTHWEST

  19        NW              

0 
NORTHWEST

  23        NE              

0 
NORTHEAST

  99        NONE            

0 
OUTSIDE ALL REGION BOUNDARIES

CODE TABLE NAME:  OWN.CODE

‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑

CODE   MAP/REPORT   
MAP  
CODE DESCRIPTION

           
  LABEL        

SYMB

----------------------------------‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑
   0        NONE            

0 
Trust Status Not Applicable Or Unknown

   1        FOR BD TRANS    
0 
State Forest Board Transfer

   2        FOR BD PURCH    
0 
State Forest Board Purchase

   3        COM SCHL/IND    
0 
Common School and Indemnity

   4        AGRIC SCH       
0 
Agricultural School

   5        UNIV‑TRANS      
0 
University ‑ Transferred

   6        CEP&RI          

0 
Charitable/Educational/Penal & Reformatory Instit.

   7        CAPITOL GRNT    
0 
Capitol Grant

   8        NORMAL SCH     
0 
Normal School

   9        ESCHEAT         
0 
Escheat

  10        SCIENTIC SCH    
0 
Scientific School

  11        UNIV‑ORIG      
0 
University ‑ Original

  12        CMNTY COLL     
0 
Community College Forest Reserve

  13        ADMIN SITE      
0 
Administrative Site

  15        BNW‑TIDE        
0 
Beds Of Navigable Waters (Abutting Tidelands)

  17        BNW‑SHORE       
0 
Beds Of Navigable Waters (Abutting Shorelands)

  20        TIDE‑1ST‑OUT    
0 
Tidelands ‑ 1st Class (Outside 4th Class Towns)

  21        TIDE‑2ND       
0 
Tidelands ‑ 2nd Class

  23        SHOR‑1ST        
0 
Shorelands ‑ 1st Class

  24        SHOR‑2ND        
0 
Shorelands ‑ 2nd Class

  25        HARBOR‑OUT      
0 
Harbor Areas (Outside 4th Class Towns)

  28        HARBOR‑INS     
0 
Harbor Areas (Inside 4th Class Towns)

  29        TIDE‑1ST‑INS    
0 
1st Class Tidelands (Inside 4th Class Towns)

  30        DOC             

0 
Washington Dept. of Corrections

CODE TALBES
CODE TABLE NAME:  OWN.CODE

‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑

CODE   MAP/REPORT   
MAP  
CODE DESCRIPTION

           
  LABEL        

SYMB

----------------------------------‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑
  31        DSHS            

0 
Washington Dept. of Social & Health Services

  32        WDF             

0 
Washington Dept. of Fisheries

  33        WDW             

0 
Washington Dept. of Wildlife

  34        WSP&RC         
0 
Washington State Parks & Recreation Commission

  35        DOT             

0 
Washington Dept. of Transportation

  36        DOT‑AERO        
0 
Washington Dept. of Transportation ‑ Aeronautics

  38        DOA             

0 
Washington Dept. of Agriculture

  40        DVA             

0 
Washington Dept. of Veterans Affairs

  41        UN REPAYMENT    
0 
University Repayment

  42        FB REPAYMENT    
0 
Forest Board Repayment

  43        LAND BANK       
0 
Land Bank

  46        CEP&RI‑TRANS    
0 
C.E.P.& R.I. Transferred

  70        UC WSP&RC       
0 
Under Contract To State Parks & Recreation

  71        UC PRIVATE      
0 
Under Contract To Private Party

  72        UC FEDERAL      
0 
Under Contract To Federal Agency

  73        UC OTHER        
0 
Under Contract To Other Agency

  74        NAP             

0 
Natural Area Preserve

  75        NRCA            

0 
Natural Resources Conservation Area

  76        DNR REC         
0 
DNR Recreation Area

  77        WPCD            

0 
Water Pollution Control Division Trust Land


CODE TABLES
CODE TABLE NAME:  DISTRICT.CODE

‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑

CODE       MAP/REPORT   
MAP  
CODE DESCRIPTION

                  LABEL        
SYMB

-------------------------------‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑‑
10       
SOUTHEAST       
0 
Southeast Region

   11       
ALPINE

0
Southeast
Alpine

   12       
KLICKITAT       
0 
Southeast
Klickitat

   13       
COLUMBIA BAS    
0 
Southeast
Columbia Basin

   14       
SNAKE RIVER     
0 
Southeast
Snake River

   20       
OLYMPIC         
0 
Olympic Region

   21       
STRAITS         

0 
Olympic
Straits

   22       
OZETTE          

0 
Olympic
Ozette

   23       
HOH             

0 
Olympic
Hoh

   24       
CLEARWATER     
0 
Olympic
Clearwater

   25       
NO GRAYS HAR    
0 
Olympic
North Grays Harbor

   40       
CENTRAL         
0 
Central Region

   41       
BLACK HILLS    
0 
Central

Black Hills

   42       
LEWIS           

0 
Central

Lewis

   43         
PACIFIC         

0 
Central

Pacific

   90       
SO PUGET        
0 
S.Puget Sound Region

   91       
SHORELINE       
0 
S.Puget Sound
Shoreline

   92       
HOOD CANAL      
0 
S.Puget Sound
Hood Canal

   95       
RAINIER         

0 
S.Puget Sound
Rainier

   96       
KING            

0 
S.Puget Sound
King

  110       
SOUTHWEST       
0 
Southwest Region

  115       
COLUMBIA        
0 
Southwest
Columbia

  116       
ST. HELENS      
0 
Southwest
St. Helens

  117       
YACOLT          
0 
Southwest
Yacolt

  190       
NORTHWEST       
0 
Northwest Region

  191       
BAKER           

0 
Northwest
Baker

  192       
CASCADE         
0 
Northwest
Cascade

  193       
ORCA

        
0 
Northwest
Orca

  230       
NORTHEAST       
0 
Northeast Region

  231       
HIGHLANDS       
0 
Northeast
Highlands

  232       
SO OKANOGAN     
0 
Northeast
South Okanogan

  233       
NO COLUMBIA     
0 
Northeast
North Columbia

  234       
ARCADIA         
0 
Northeast
Arcadia

  999       
NONE            

0 
Outside all District boundaries

