Using EXCEL SOLVER

SOLVER can be used to solve non-linear (and linear) problems. It has options to the find the values for a set of cells to minimize (or maximize) the value in a cell or so that the value in a cell equals a pre-specified value (usually 0). Note that this cell must contain a formula and not a value.

Important note: Before you use SOLVER you need to make sure that it is installed. If it is not installed, go to the Tools menu, select the Add-Ins menu-item, click the Solver Add-In box and select OK.

The example problem involves fitting a model of the form  to the population counts for the bowhead population. The function minimized to find the values for  and  is .  The calculations to set SS are in cells A2:D18 of the sheet SOLEX.XLS.  To start SOLVER, Select Tools and then Solver.

There are several key options in the Solver dialogue box that need to be set to use SOLVER successfully.

- Type a reference to the cell that contains the value of the function to be minimized (maximized or set to a specific value) in the “Set Target Cell” field.

- Type references to the cells the values of which are to be varied to minimize the target cell in the By Changing Cell” field. You can type cells names rather than cell references here.

- It is possible to bound values. To do this, click “Add”; a new dialogue box (see below) is opened that allows you to specify the bounds. In this case, you want to force the intercept (cell B16) to be larger than zero (to avoid taking the logarithm of a negative number).

Once you have entered the first constraint, select “Add” and you will be presented with the opportunity to add another constraint. Once you have added all the constraints, select “Cancel” and you will be returned to the original Solver dialogue box.

In principle, you can select “Solve” at this point and SOLVER should be able to find the solution. If you do this, you will get the following dialogue box:

If you like the solution you got from SOLVER, click OK.

The performance of SOLVER can be improved (often markedly) by making use of some of the advanced features available in the “Options” dialogue box (you can get to this dialogue box by selecting “Options” in the “Solver Parameters” dialogue box).

There are many fields in this dialogue box. The options I use regularly are:

- “Use Automatic Scaling”. This rescales the values in the cells when conducting the minimization (you won’t see this happening but it does happen). This option should always be selected as it often substantially improves the performance of SOLVER.

- “Assume Non-Negative”. This option places a lower bound of 0 on all parameters not bounded using the “Add constraint” dialogue box.

- “Max Time” and “Iterations”. Increase the values in these cells if you encounter problems that are not converging. You should not see the dialogue box below in this course (doing so would mean that you made a mistake somewhere in the spreadsheet).

It often a good idea when using SOLVER to try different initial values in the cells that are varied to solve the problem. Also, I usually run SOLVER multiple times just in case it stopped before it solved the problem (this happens remarkably frequently). SOLVER can be called from within Visual Basic.  We will be seeing a macro for doing this later in the course.