Here is the study guide for the first exam.  For those of you who haven’t taken a class from me I will typically ask questions based on what is asked in this document.  For this class, I will also look back at the questions from the homework so it is important that you also look at these.

# Introduction and Raw Material (Lecture 1)

- I hope you learned a little bit about the history of papermaking in the first part of this lecture.  Here are a couple of questions regarding what I hoped you learned in this section.

  - Before paper was made from wood, what was it made from?

  - When were the first chemical pulping processes developed?

- There are large differences in raw material use for pulp production in the United States based on geographic location.  I expect that you would know the following:

  - Are hardwoods or softwoods most used in the West?  How about roundwood versus residual?

  - Are hardwoods or softwoods most used in the Southeast and South Central?  How about roundwood versus residual?

  - Are hardwoods or softwoods most used in the Northeast and North Central?  How about roundwood versus residual?

  - Why are there these differences in use patterns?

- When material (wood/chips/whatever) comes into a mill there are a number of things which happen such as debarking, chipping, washing, screening, testing etc.  Why are these important?

- Physical Measurements

  - Moisture content: Do you know the equations for moisture content green and oven dry?  Which one is used in the pulp and paper business

  - Density:  Why is density important?

  - Why is chip size important?

  - What problems do debris cause?

- Wood degradation:  Why is this process a problem for pulp manufacture?

  - There are a variety of variables that affect the decay processes that go on in a chip pile.  Why are the following

    - Time in the pile

    - Moisture

    - Temperature

    - Shape of the pile

# Mechanical Pulping: (Lecture 2)

- What is different between the raw materials used for stone groundwood and for refiner pulps?

  - What is important about the raw materials used in both processes in terms of age, species, cleanliness, etc?

- Can you list in general terms the properties of mechanical pulps?

- Can you describe the operation of a stone groundwood operation?

  - Why are there burrs on the stone?

  - How often are these burrs “sharpened”?

  - Why is water used in this process?

- Can you explain the mechanism of the removal of fibers?

  - What effect does heat or chemicals have on this process?

  - What portion of the cell wall does separation occur?

  - What is the effect of getting the fibers too hot?  Where does separation occur under these conditions.  What effect does this have on the resulting fibers?

- Can you name the different type of refiner processes?

- Can you draw a basic flowsheet for a refiner plant?

  - Why are chips washed before refinning?

  - Why is there more than one refiner in series in the process flow sheet?

  - What is the purpose of a latency tank?

- Can you describe a refiner plate?

- Can you name the conditions used for each refining process (RMP/TMP/CTMP/CMP) in terms of:

    - Is there a preheating time?  At what temperature?

    - Are there chemicals used?  What are they?  How long a treatment?  At what temperature do they chemically treat?

    - Why does the CTMP produce a product superior to RMP and TMP?

- How is endpoint determined in mechanical pulping?

# Mechanical Pulp Bleaching (Lecture 3)

- Can you explain why mechanical pulp is not as bright as chemical pulp

  - Why does mechanical pulp turn yellow on exposure to light?

    - What types of chemical structures are responsible for this yellowing and for the reduced brightness?

- What are the 2 systems used to bleach mechanical pulps?

- Can you provide a basic discussion of how each of these systems work chemically?

- Can you provide a description of the system variables used in both these systems and how they affect the final product?

  - What chemicals are used and why?

  - What other variables are important?

  - Which system is more likely to suffer through color reversion?

  - Why are chelating agents added to the oxidative system?

- Which of these 2 systems is the most efficient?

# Introduction to Chemical Pulping (Lecture 4)

- Chip quality issues: there are a lot of issues related to chip quality mentioned in my lectures.  Here are some issues that I find important.

  - Chip Thickness:  Why is chip thickness such an important variable?

  - Why is chip moisture content important for pulping?

  - How does wood density affect production, and pulp qualities?  Are hardwoods or softwoods typically denser?

  - Why is it not a good idea to pulp forest thinnings with chips from mature trees?

  - What problems does heartwood pose for pulping and for pulp properties?

  - Reaction wood:  How does compression wood and tension wood affect pulp?

  - How does the use of decayed wood affect pulp properties?

- Batch reactors

  - Typically how large are these reactors?

  - Typically how many batch digesters does a typical kraft mill have?

- Can you describe the methods used to pack a digester?

  - Which methods work best?

  - Why do you want improved packing?

- Why would you want to presteam?

- Can you describe the methods used to heat a digester?

  - Can you name the benefits of both?

- Chip impregnation

  - What is the difference between penetration and diffusion of liquor? In general terms what variables affect each process?

- General Kraft Pulping Process:

  - Why is the heating time usually longer than time at temperature?

  - Why is there gas relief?  What gasses are vented?

  - Can you describe the process of blowing down the digester?

    - Why do the chips turn into fibers at this point? (2 reasons)

  - What is displacement pulping?  Why is this done?

# Continuous Reactors (Lecture 5)

This was a pretty short lecture.  There are several facts I wanted you to understand.

- There is more chemical pulp produced using continuous reactors than batch reactors.

- Chips move continuously through these reactors; all the same steps that occur in a batch system occur in a continuous reactor.

  - Can you name the different zones in a continuous reactor?

  - Can you provide a rough temperature profile of a reactor?

  - Why is there different direction of flow of liquor in different sections?

  - Roughly how long is the retention time in each section?

  - Do you know the steps involved in getting chips into a continuous digester?

    - Why presteam?

    - How are the chips charged into the reactor?

      - Why is excess white liquor used?

  - Why is black liquor used to “wash” the chips if what the washing system is supposed to do is remove black liquor?  (Question thanks to Paul and Joe)

- Important: Can you compare the benefits of both the batch and continuous systems?

# Kraft Pulping Chemicals (Lecture 6)

- White liquor versus black liquor: In simple terms, can you describe what these are?

- You need to be able to name (and know the chemical structure) of the 6 components of white liquor

  - Can you identify their source (origin)?

  - Can you identify which ones are active reactants?

  - Roughly how much NaOH and Na2S are found in black liquor

  - Which has more dissolve organic material, black or white liquor?

- White liquor equations

  - You will need to know the equations for:

    - Total Alkali

    - Effective Alkali

    - Active Alkali

    - Sulfidity

    - Reduction Efficiency

    - Causticizing Efficiency

- The Na2O issue

  - You need to be able to convert any sodium species in the world to a Na2O basis.  This is something you need to be able to do in your sleep (like drawing the structure of vanillin).

- You need to know reaction equation for when Na2S is added to water?

  - Why isn’t there more H2S produced in this reaction?

- Let me make this very simple for you.  In homework #6 and on the in class problems I asked you to make a variety of calculations.  You need to understand how the answers to these questions were obtained.  You also need to be able to work these types of problems without a lot of thinking time.  There will be questions like these on the exam and in real life.

  - Can you work with moisture contents to determine how much dry wood you have?

  - Do you understand the concept of liquor to wood ratio?

  - Can you convert from g/liter to charge (% on wood)

  - Can you convert from g/l Na2O to moles/liter of the actual chemical

  - Do you understand the concept that the amount of OH- in solution is dependent upon NaOH and Sulfide

  - Basically make sure you understand the questions from Homework #6…..don’t just look at the questions; understand what you are doing.