- Which statement best describes black liquor? (1 point)

  - A dilute solution containing mostly inorganic solids at a pH~14

  - A dilute solution containing mostly organic solids at a pH~14

  - A dilute solution containing mostly organic solids at a pH>12

  - A concentrated solution containing mostly inorganic solids at a pH>12

  - A concentrated solution containing mostly organic solids at a pH~14

  - A concentrated solution containing mostly inorganic solids at a pH~14

- The inorganic in the highest concentration in black liquor is: (1 point)

  - Sodium hydroxide

  - Sodium sulfide

  - Sodium carbonate

  - Sodium sulfate

- What is the minimum concentration black liquor is evaporated to prior to burning? (1 point)

  - >40%

  - >60%

  - >80%

- What is the maximum concentration a black liquor can be evaporated to using direct heating? (1 point)

  - 65%

  - 70%

  - 80%

- What is the name of the process by which organics in the recovery furnace react with oxygen forming CO2, CO, and heat? (1 point)

  - Combustion

  - Pyrolysis

  - Reduction

- What is the name of the process by which organics are converted by heat to volatile hydrocarbon type substances? (1 point)

  - Combustion

  - Reduction

  - Pyrolysis

- What is the process by which sodium sulfate is converted to sodium sulfide in the char?

  - Combustion

  - Pyrolysis

  - Reduction

- When the smelt is added to water, what is formed? (1 point)

  - White Liquor

  - Green Liquor

- What is slaking? (1 point)

  - The addition of smelt to water

  - The conversion of sodium carbonate to calcium carbonate

  - The conversion of calcium carbonate to CaO

  - The conversion of CaO to Ca(OH)2

- What happens during causticizing? (1 point)

  - Sodium sulfate is converted to sodium sulfide

  - Calcium carbonate is converted to CaO

  - Sodium carbonate is converted to NaOH

- Which of the following is in highest concentration in green liquor? (1 point)

  - Sodium hydroxide

  - Sodium sulfate

  - Sodium carbonate

  - Sodium sulfide

- What is the function of the lime kiln? (1 point)

  - Conversion of sodium carbonate to sodium hydroxide

  - Conversion of CaO to Ca(OH)2

  - Conversion of CaCO3 to CaO

  - Conversion of sodium sulfate to sodium sulfide