- Ozone bleaching is carried out under which of the following conditions? (1 point)

  - Acidic

  - Neutral

  - Alkaline

- True or False:  Ozone (O3) reacts rapidly with carbohydrates. (1 point)

  - True

  - False

- True or false: Ozone will cleave the olefinic linkage in stilbenes. (1 point)

  - True

  - False

- Why is ozone bleaching run at low temperatures? (1 point)

  - To protect carbohydrates by limiting the production of oxygen radicals from ozone.

  - To slow down the reaction rate to allow diffusion to keep up.

  - To limit alkaline induced yellowing of the pulp.

- It is very important to wash pulp thoroughly after oxygen delignifcation if the next stage is an ozone stage.  Why is this? (1 point)

  - Oxygen bleaching is alkaline while ozone bleaching is acidic.

  - A large amount of ozone will be consumed by reaction with dissolved organics (carry over) in the pulp.

  - Dissolved oxygen in the bleaching liquor can cause ozone to degrade to radicals.

  - a&b

  - All of the above.

- If you are peroxide bleaching at 95 C, are you delignifying or brightening? (1 point)

  - Delignifying

  - Brightening

- In solution, peroxide can break down to a variety of compounds.  Which of the following is most responsible for delignification? (1 point)

  - Hydrogen peroxide

  - Hydroperoxy anion

  - Superoxide

  - Hydroxyl radical

- Which is the desired peroxide compound for pulp brightening? (1 point)

  - Hydrogen peroxide

  - Hydroperoxy anion

  - Superoxide

  - Hydroxyl radical

- Under what conditions is peroxide bleaching carried out? (1 point)

  - Acidic

  - Neutral

  - Alkaline

- Which of the following is not added to stabilize peroxides? (1 point)

  - EDTA

  - Magnesium sulfate

  - DPTA

  - Sodium silicates