- The process of producing groundwood fibers consists of: (1 point)

  - The ripping off of fibers by a stone.

  - The peeling off of fibers by the stone.

  - The loosening of fibers by compression and decompression by the stone.

  - The removal of fibers by the heat induced removal of lignin.

- True or false: Chips of any age are used in refiner processes. (1 point)

  - True

  - False

- True or false: The stones used in groundwood contain patterns (burrs) which must be sharpened every 2-3 weeks. (1 point)

  - True

  - False

- True or false: Higher temperatures are avoided in stone groundwood pulping because of fears of fiber darkening. (1 point)

  - True

  - False

- True or false: There are more burrs per square inch in the outer portion of a refiner plate than on the inner portion. (1 point)

  - True

  - False

- In which refiner process are the chips preheated before refining but not treated with chemicals? (1 point)

  - RMP

  - TMP

  - CMP

  - CTMP

- In which refiner process are the chips treated with a low (2-5%) sodium sulfite treatment before refining? (1 point)

  - RMP

  - TMP

  - CMP

  - CTMP

- What does NSSC stand for? (1 point)

  - Neutral sulfite semi chemical

  - National Starch sizing compound

  - Nitrous sulfite semi chemical

  - Neutral sulfite semi continuous

- True or false: A high Canadian Standard Freeness means that a refiner pulp has been slightly refined? (1 point)

  - True

  - False

- The major reason that paper made from mechanical pulp has high opacity is:  (1 point)

  - The retained lignin reflects light

  - The high amounts of fines scatter light.

  - The highly torn up (frayed) fibers scatter light.

- Which of the following is not an attribute of mechanical pulp? (1 point)

  - Forms a good printing surface.

  - Relatively weak product.

  - Relatively small distribution of fiber sizes.

  - High opacity.

- In your opinion, which of the following is the highest cost item in the production of mechanical pulp in today's market? (1 point)

  - Wood

  - Chemicals

  - Electricity

  - Labor

- Which of the following systems would most likely give you a fiber of highest strength and brightness? (1 point)

  - CTMP

  - Groundwood

  - RMP

  - TMP