- Which of the following best represents the typical height of a Kamyr digester? (1 point)

  - 10-20 meters

  - 50-70 meters

  - 100-120 meters

  - 200-250 meters

- What is the range of capacities of continuous digesters? (1 point)

  - 100-500 a.d. metric t/day

  - 200-1000 a.d. metric t/day

  - 500-1500 a.d. metric t/day

  - 200-2000 a.d. metric t/day

- Which of the following best describes the presteaming process? (1 point)

  - System used to heat chips and expel air and volatiles over a 20-30minute period

  - System used to heat chips and expel air and volatiles over a 2-3 minute period

  - System used to heat chips and impregnate chips with white liquor over a 10-15 minute period.

- Why is an excess of white liquor added to the chips in the high pressure feeder? (1 point)

  - To assure impregnation.

  - To convey the chips.

  - To eliminate trapped air.

- How long do chips typically reside in the impregnation zone? (1 point)

  - 10-20 minutes

  - 30-40 minutes

  - 45 -60 minutes

  - 90-120  minutes

- What is the temperature range in which chips undergo impregnation? (1 point)

  - 70-90 C

  - 115-125 C

  - 130-145 C

  - 150-170 C

- What is the maximum temperature range at which chips are typically cooked in a continuous kraft digester? (1 point)

  - 100-120 C

  - 120-140 C

  - 150-170 C

- Depending on the mill, how long do chips reside in the column? (1 point)

  - 1-2 hours

  - 2-6 hours

  - 3-8 hours

  - 5-12 hours

- Why does the concentration of NaOH vary from the top to the bottom of a continuous kraft digester? (1 point)

  - Consumption of NaOH during delignification.

  - Washing liquor dilutes the stream.

  - Different concentrations of liquor are charged into the digester in different parts of the system.

  - All of the above.

- The chips (fibers) are washed in the continuous kraft digester with: (1 point)

  - White liquor

  - Black liquor

  - Water

  - a & b

  - b & c

  - All of the above

- True or false: Monthly production rates in mills using batch digesters is more consistent than those using continuous digesters. (1 point)

  - True

  - False

- True or false: Continuous digester systems use less energy than batch systems. (1 point)

  - True

  - False